МИНИСТЕРСТВО НАУКИ И ВЫСШЕГО ОБРАЗОВАНИЯ РОССИЙСКОЙ ФЕДЕРАЦИИ

федеральное государственное автономное образовательное учреждение высшего образования

«САНКТ-ПЕТЕРБУРГСКИЙ ГОСУДАРСТВЕННЫЙ УНИВЕРСИТЕТ 
АЭРОКОСМИЧЕСКОГО ПРИБОРОСТРОЕНИЯ»

КАФЕДРА №23

КУРСОВАЯ РАБОТА 
ЗАЩИЩЕНА С ОЦЕНКОЙ

РУКОВОДИТЕЛЬ

| канд. техн. наук, доцент |  |  |  | В. А. Ненашев |
| --- | --- | --- | --- | --- |
| должность, уч. степень, звание |  | подпись, дата |  | инициалы, фамилия |

ПОЯСНИТЕЛЬНАЯ ЗАПИСКА К КУРСОВОЙ РАБОТЕ

РАЗРАБОТКА ПРОГРАММЫ ФОРМИРОВАНИЯ ШИМ ДЛЯ УПРАВЛЕНИЯ ДВИГАТЕЛЯМИ

по дисциплине: МИКРОПРОЦЕССОРНЫЕ ИНФОРМАЦИОННО-ИЗМЕРИТЕЛЬНЫЕ И УПРАВЛЯЮЩИЕ УСТРОЙСТВА

РАБОТУ ВЫПОЛНИЛ

| СТУДЕНТ гр. № | 2337М |  |  |  | А. В. Разваляев |
| --- | --- | --- | --- | --- | --- |
|  |  |  | подпись, дата |  | инициалы, фамилия |

Санкт-Петербург 2025

СОДЕРЖАНИЕ

ВВЕДЕНИЕ	3

1 Управление двигателями	4

1.1 Управление бесколлекторными двигателями постоянного тока	5

1.2 Управление синхронными и асинхронными двигателями	6

2 Разработка программы формирования ШИМ	7

2.1 Методы формирования ШИМ	8

2.2 Дистанционное управление программой	10

2.3 Локальное управление программой	11

2.4 Управление параметрами ШИМ и логирование данных	13

3 Демонстрация работы программы	14

4 Тестирование программы на бесколлекторном двигателе	15

ЗАКЛЮЧЕНИЕ	16

ПРИЛОЖЕНИЕ А	17

ПРИЛОЖЕНИЕ Б	38

ПРИЛОЖЕНИЕ В	50

ПРИЛОЖЕНИЕ Г	69

ПРИЛОЖЕНИЕ Д	75

ПРИЛОЖЕНИЕ Е	80

# ВВЕДЕНИЕ

Современные технологии управления электрическими двигателями играют ключевую роль в развитии промышленности и автоматизации. В последние годы существенно возрос интерес к бесколлекторным и асинхронным электродвигателям, что связано с их высокой эффективностью и надежностью. В данной работе представлено программное обеспечения для управления такими двигателями.

Объектом исследования являются бесколлекторные и асинхронные двигатели, которые благодаря своей конструкционной особенности и возможностям работы с минимальными потерями энергии становятся все более востребованными в различных отраслях.

В работе также анализируются достоинства платформы STM32, использующейся для разработки программного обеспечения, что обеспечивает высокую производительность и эффективность за счет особенностей архитектуры и возможности легкого программирования. Все эти аспекты представляют значительный интерес для предприятий, стремящихся внедрить передовые технологические решения в свои производственные процессы, и подтверждают необходимость дальнейшего изучения и применения данных технологий в управлении электродвигателями.

Целью работы является разработка программного обеспечения для управления ШИМ, записи логов и обеспечения базового интерфейса взаимодействия. Задачи:

- разработать программные модули для генерации и управления ШИМ;

- реализовать механизм логирования данных;

- реализовать интерфейс для взаимодействия пользователя с системой;

- провести тестирование разработанных модулей.

# 1 Управление двигателями

Современные электрические двигатели широко применяются в промышленности, транспорте и бытовой технике. Их управление позволяет регулировать скорость вращения, момент, направление движения и другие параметры работы, что делает их универсальным инструментом для автоматизации. Основные подходы к управлению двигателями зависят от типа двигателя. В данной работе будет рассмотрено разомкнутое управление, бесколлекторным и синхронным или асинхронным двигателем.

Следует отметить, что схема управления двигателем электрически развязана от схемы логики микроконтроллера. Для силовой части двигателя, отвечающей за непосредственное питание и коммутацию обмоток, используются отдельные платы (инвертор). Принципиальная схема простейшего инвертора одной фазы приведена на рис. 1.

Рисунок 1 – Схема простейшего инвертора (1 фаза)

Инвертор преобразует постоянный ток в переменный за счет модуляции ширины импульсов управления ключами VD.

В основе работы инвертора лежит схема на силовых транзисторах (обычно IGBT или MOSFET), управляемых контроллером, который определяет последовательность и длительность коммутации фаз. В рамках исследования рассматривается только принцип формирования сигналов ШИМ, необходимых для управления транзисторами силовой схемы. Проектирование этой силовой части не является целью данной работы.

# 1.1 Управление бесколлекторными двигателями постоянного тока

Управление бесколлекторным двигателем основывается на последовательном включении обмоток статора, чтобы создать вращающееся магнитное поле, которое взаимодействует с постоянными магнитами ротора. Этот процесс обеспечивает вращение ротора без механических щеток.

На рис. 2 изображен принцип управления трехфазным бесколекторным двигателем. В каждый момент времени активны две обмотки: через одну подается ток, а через другую он стекает. Третья обмотка остается отключенной. Для трехфазного двигателя существует шесть активных состояний, которые формируют одну электрическую фазу вращения. Эти состояния переключаются последовательно в зависимости от позиции ротора.

Рисунок 2 – Состояния бесколлекторного двигателя

Частота переключения этих состояний определяет скорость вращения двигателя. Чем выше частота, тем быстрее вращается ротор.

ШИМ используется для управления мощностью, передаваемой на фазы двигателя, обеспечивая плавность работы и минимизацию потерь. Изменяя скважность сигнала ШИМ, можно контролировать среднее напряжение, подаваемое на обмотки (рис. 3).

Рисунок 3 – График ШИМ для управления мощностью бесколлекторного двигателя

# 1.2 Управление синхронными и асинхронными двигателями

Для синхронных/асинхронных двигателей формируется синусоидальное напряжение с помощью инвертора, и частота этого напряжения определяет скорость вращения ротора. Для этого необходимо менять скважность ШИМ по синусоидальному закону. На рис. 4 изображен принцип формирования сигнала ШИМ двух ключей одного плеча (u1, u2), для получения синусоидального тока.

Рисунок 4 – График ШИМ для формирования синусоидального напряжения

# 2 Разработка программы формирования ШИМ

Выбор платформы для разработки программного обеспечения представляет собой важнейший этап в процессе создания эффективных систем управления двигателями. Одной из наиболее подходящих платформ для разработки программного обеспечения, включая алгоритмы формирования широтно-импульсной модуляции (ШИМ), является STM32. Возможность гибкой настройки таймеров для генерации ШИМ-сигналов позволяет реализовывать различные методы управления двигателями с высокой точностью.

Кроме того, STM32 поддерживает разнообразные интерфейсы для связи с другими устройствами, включая UART, SPI и CAN, что важно для обмена данными в распределенных системах. Поддержка множества встроенных функций, таких как цифровая обработка сигналов (DSP) и математические операции на аппаратном уровне, значительно ускоряет вычисления, требующиеся для алгоритмов управления.

В результате использования платформы STM32 в сочетании с оптимизированным программным обеспечением создается надежная и мощная система управления, способная удовлетворить требования различных промышленных приложений.

# 2.1 Методы формирования ШИМ

Для формирования ШИМ были использованы таймеры. Они являются основным инструментом для формирования сигналов ШИМ в микроконтроллерах STM32, позволяя генерировать импульсы с заданной частотой и коэффициентом заполнения.

К таким параметрам относятся частота работы таймера, коэффициент заполнения и длительность импульсов. Эти параметры задаются через конфигурацию регистров таймера, таких как предделитель (Prescaler), регистр автоматической перезагрузки (ARR) и регистр сравнения (CCR). Настройка этих параметров позволяет добиться необходимой точности и стабильности ШИМ сигнала, что особенно важно в системах управления двигателями.

При настройке таймеров на STM32 для генерации ШИМ важно учитывать практические аспекты, такие как синхронизация таймеров для управления несколькими каналами и использование функций dead-time, предотвращающих короткое замыкание в полумостовых схемах. Т.к. встроенной функции dead-time в STM32 нет, то она реализуется программно. Эти функции имеют особое значение при работе с мощными двигателями, где требуется высокая надежность.

В программе используется 3 таймера для формирования ШИМ: TIM4, TIM3 и TIM2. Таймер TIM4 является ведущим и по нему в режиме синхронизации запускаются таймеры TIM3, TIM2. Настройка частоты и скважности таймера зависит от значений, записанных в регистрах Modbus, которые описаны в главе 2.2.

Для каждого плеча используется один таймер с двумя каналами ШИМ.

Скважность ШИМ обновляется каждый период, по прерыванию таймера TIM4. В прерывании, в зависимости от выбранного режима вызывается соответствующая функция для управления скважностью.

Поддерживаются следующие режимы:

- Постоянный одноканальный ШИМ (на любом из 6 каналов)

- Постоянный мостовой однофазный ШИМ

- Постоянный мостовой трехфазный ШИМ

- Синусоидальный одноканальный ШИМ (на любом из 6 каналов)

- Синусоидальный мостовой однофазный ШИМ

- Синусоидальный мостовой трехфазный ШИМ

Для плавного изменения параметров в программе используется ПИ-регулятор, из библиотеки CMSIS.

Т.к. данная программа реализует разомкнутое управление без обратной связи, то выход ПИ-регулятора заведен на вход. Также реализована сатурация, для ограничения скорости изменения скорости – функция.

В результате был написан модуль ШИМ (pwm.c/pwm.h), который реализует всю его работу. Он содержит функции для двух основных режимов ШИМ:

- Постоянный ШИМ (DC) – функция PWM_DC_Bridge_Mode().

- Синусоидальный ШИМ (SINE) – функции и PWM_Sine_Bridge_Mode().

Эти режимы обеспечивают формирование различных ШИМ для управления двигателями.

Так же есть режим одноканального ШИМ, который можно использовать для двигателей постоянного тока или для тестирования переключения отдельных транзисторов – функция PWM_SingleChannel_Mode().

Полный листинг данного модуля приведен в приложениях А, Б.

# 2.1 Дистанционное управление программой

Для дистанционного управления программой было решено использовать протокол Modbus

Протокол Modbus – самый распространенный промышленный протокол для взаимодействия между полевым уровнем (измерительных устройств – датчиков) и контроллерным уровнем. Является стандартом и поддерживается почти всеми производителями промышленного оборудования.

Благодаря универсальности и открытости, стандарт позволяет интегрировать оборудование разных производителей. Modbus используется для сбора показания с датчиков, управления реле и контроллерами, мониторинга, и т.д.

В протоколе Modbus данные организованы в виде 16-битных регистров и однобитных коилов, что делает его удобным для передачи и обработки информации в системах управления. Регистры используются для хранения числовых значений, таких как параметры настройки или данные измерений, а коилы представляют собой логические состояния, которые могут быть использованы для управления или индикации. Такое разделение позволяет эффективно структурировать данные, обеспечивая простоту интеграции в системы автоматизации.

Передача данных по Modbus на STM32 реализуется через интерфейс UART.

Также при использовании Modbus важно правильно распределить регистры и коилы. Регистры могут хранить параметры ШИМ, такие как частота и коэффициент заполнения, в то время как коилы можно использовать для переключения режимов работы.

В итоге была реализована следующая структура данных Modbus:

Параметры ШИМ записываются в регистры и, если настройки обновились, таймеры STM32 перенастраиваются в соответствии с новыми настройками:

Регулируемые параметры:

- Частота синуса (Регистр №20000),

- Скважность ШИМ (Регистр №20001),

- Частота ШИМ (Регистр №20002),

- Максимальная длительность одного импульса (Регистр №20003),

- Минимальная длительность одного импульса (Регистр №20004),

- Deadtime для переключения ключей между каналами (Регистр №20005).

Режим работы ШИМ записываются в коилы и, в соответствии с которыми вызываются необходимые функции для формирования ШИМ:

- Постоянный или синусоидальный ШИМ (Коил №17),

- Мостовой ШИМ (Коил №18),

- Трехфазный ШИМ (Коил №19),

- Полярность ШИМ (Коил №20),

- Используемый канал ШИМ в одноканального режиме (Коил №21-23).

# 2.2 Локальное управление программой

Также было решено добавить локальное управление, через энкодер с кнопкой и дисплей.

Т.к. STM32 имеет аппаратную поддержку I2C и SPI, то удобно будет использовать дисплеи именно с такими интерфейсами

Интерфейс I2C представляет собой двухпроводной протокол передачи данных, использующий линии SDA (Serial Data) и SCL (Serial Clock). Он позволяет нескольким устройствам работать на одной шине, передавая данные в последовательном формате с подтверждением на каждом байте.

Дисплеи, работающие по I2C, имеют встроенный контроллер, который принимает команды и преобразует их в параллельный интерфейс дисплея. Благодаря этому, для управления дисплеем требуется минимум соединений, что особенно полезно при использовании микроконтроллеров с ограниченным числом выводов.

Такие дисплеи, как правило, символьные и имеют размер 2 строки 16 символов (LCD1602) или 4 строки 20 символов (LCD2004). В данной работе использовался дисплей LCD1602.

Интерфейс SPI представляет собой стандарт для высокоскоростного обмена данными между микроконтроллерами и периферийными устройствами. Принцип работы SPI основывается на синхронной передаче данных, где используются четыре сигнальных линии: MOSI (Master Out Slave In), MISO (Master In Slave Out), SCK (Serial Clock) и SS (Slave Select).

Дисплеи с интерфейсом SPI, такие как LCD12864, обеспечивают быструю передачу данных благодаря своей архитектуре, что позволяет обновлять изображение на экране практически мгновенно. Также они, как правило, графические и работают на уровне отдельных пикселей, а не фиксированных символов, что делает их более гибкими в плане отображаемой информации. Также они могут быть больше по размерам (напр. LCD12864 размером 128x64 пикселей), что позволяет выводить гораздо больше информации. Тем не менее, дисплеи SPI имеют и свои недостатки: они требуют большего числа проводов по сравнению с интерфейсами, такими как I2C, что может усложнять разводку схемы.

Энкодеры представляют собой устройства, предназначенные для преобразования механического движения в электрические сигналы, которые могут быть интерпретированы системой управления. Основной принцип их работы заключается в регистрации изменений положения объекта, будь то угловое или линейное перемещение. Это достигается благодаря использованию датчиков, которые фиксируют изменения положения относительно начальной точки. В зависимости от конструкции и типа энкодера, такие сигналы могут быть либо импульсными, либо представлять собой закодированную информацию о текущем положении.

При разработке программы для задания параметров было решено использовать угловой энкодер с кнопкой. Микроконтроллер STM32 предоставляет специализированные режимы таймеров для работы с энкодерами, что позволяет эффективно считывать угол поворота энкодера.

В результате был написан модуль для интерфейса программы (interface.c/interface.h), который содержит функции для взаимодействия пользователя с программой. Он состоит из энкодера с кнопкой и дисплея. Основные функции:

- Обработка ввода от энкодера, включая переключение режимов и настройку параметров (грубая и тонкая настройки) – функция Update_Encoder(),

- Обновление информации на дисплее, отображение текущих параметров системы – функция Update_Encoder().

Энкодер с кнопкой позволяет пользователю выбирать и настраивать параметры системы:

- Однократное нажатие кнопки – выбор параметра для регулирования,

- Прокрутка – при выбранном параметре его регулирование, иначе прокрутка меню параметров,

- Двукратное нажатие кнопки – переключение между выбором режима ШИМ и регулированием его параметров.

Дисплей обеспечивает обратную связь, отображая текущие настройки и состояния. Есть поддержка LCD дисплея по интерфейсу I2C (LCD1602) и SPI (LCD12864).

Полный листинг данного модуля приведен в приложениях В, Г.

# 2.3 Управление параметрами ШИМ и логирование данных

Т.к. таймер ШИМ имеет очень высокую и непостоянную частоту, то было решено добавить еще один управляющий таймер, который будет перенастраивать ШИМ и а также заниматься логированием данных.

В результате был написан модуль для контроля программы (control.c/control.h), который настраивает управляющий таймер на определенную частоту.

В прерывании этого таймера проверяются настройки, заданные в Modbus и если они обновляются – перенастраиваются таймеры).

Модуль также содержит функции для сбора и записи данных о работе ШИМ:

- Запись текущих параметров в массив логов – функция Fill_Logs_with_Data(),

- Обновление параметров логирования (частота логирования, объем логированных данных) – функция Update_Params_For_Log().

Логи включают в себя следующие данные:

- Текущую скважность ШИМ

- Визуализация скважности ШИМ (в трех периодах)

- Счетчик (пила) для проверки корректно ли работает логирование

Полный листинг данного модуля приведен в приложениях Д, Е.

# 3 Демонстрация работы программы

Видео с демонстрацией управления ШИМ и считыванием логированных данным с МК через Modbus, (приложение на ПК) приведено здесь: https://disk.yandex.ru/i/7S2R2Ckg_3fXMw.

Видео с демонстрацией переключения режимов ШИМ с помощью энкодера и отображения текущего режима на дисплее LCD12864 приведено здесь: https://disk.yandex.ru/i/pxXePqLXaXEKkA.

Видео с регулированием параметров ШИМ с помощью энкодера и отображения текущих параметров на дисплее LCD1602 приведено здесь: https://disk.yandex.ru/i/xKvLAbo4s5YlPw.

График работы ПИ-регулятора приведен на рис. 5.

Рисунок 5 – Демонстрация регулирования скважности ШИМ (черная линия):

набор и сброс частоты

# 4 Тестирование на бесколлекторном двигателе

Данная программа также была протестирована на реальном бесколлекторном двигателе. Видео демонстрирующее его управление приведено здесь (разгон до 70 об/с при частоте ШИМ 2 кГц): https://disk.yandex.ru/i/eEsR5Wqq7ed-LA.

Максимальная частота вращения до которой удалось разогнать двигатель составила 400 об/с (при частоте ШИМ 40 кГц).

# ЗАКЛЮЧЕНИЕ

В ходе данной курсовой работы была разработана программа формирования ШИМ для управления различными типами двигателей с использованием микроконтроллера STM32. Разработанное ПО успешно выполняет поставленные задачи, обеспечивая управление ШИМ-сигналами и логирование данных в реальном времени.

Особое внимание уделено программной реализации формирования ШИМ с использованием таймеров STM32, что позволило продемонстрировать их практическую применимость. Кроме того, в работе были рассмотрены дополнительные модули, такие как интерфейс Modbus, дисплеи для отображения информации и энкодер для выбора режимов работы. Эти модули обеспечивают взаимодействие пользователя с системой и повышают её функциональность

Практическая значимость данной работы заключается в создании универсального подхода к управлению двигателями с использованием ШИМ. Реализованная программа может быть адаптирована для различных типов двигателей, что делает её полезной для применения в системах автоматизации, робототехнике и других областях. Кроме того, использование интерфейса Modbus и дисплеев для вывода информации позволяет интегрировать разработанную систему в более сложные решения, обеспечивая удобство управления и мониторинга

Программа обладает модульной структурой, что облегчает добавление новых функции. Например, добавление датчиков скорости для формирование обратной связи. Или добавление векторного управление двигателями.

# ПРИЛОЖЕНИЕ А

Листинг pwm.c

/** ************************************************************************** * @file pwm.c * @brief Модуль для реализации PWM. *************************************************************************/ #include "pwm.h" #include "rng.h" PWM_HandleTypeDef hpwm1; PWM_SlaveHandleTypeDef hpwm2; PWM_SlaveHandleTypeDef hpwm3; uint32_t sin_table[SIN_TABLE_SIZE_MAX]; unsigned ActiveChannelSHDW_Master; float DeadTimeCnt_Master; unsigned ActiveChannelSHDW_Slave2; float DeadTimeCnt_Slave2; unsigned ActiveChannelSHDW_Slave3; float DeadTimeCnt_Slave3; float shiftPHASE1= 0.6666666666; float shiftPHASE2= 0.3333333333; /** * @brief First set up of PWM Two Channel. * @details Первый инит ШИМ. Заполняет структуры и инициализирует таймер для генерации синуоидального ШИМ. * Скважность ШИМ меняется по закону синусоиды, каждый канал генерирует свой полупериод синуса (от -1 до 0 И от 0 до 1) * ШИМ генерируется на одном канале. * @note This called from main */ void PWM_Sine_FirstInit(void) { hpwm1.sPWM_Config.PWM_Mode = (PWM_ModeCoilsTypeDef *)&coils_regs[0]; hpwm1.sPWM_Config.PWM_Settings = (PWM_ModeRegsTypeDef *)&pwm_ctrl[0]; // PWM RAMP INIT hpwm1.pDuty_Table_Origin = SIN_TABLE_ORIGIN; // SINE RAMP PARAMS hpwm1.hramp[PWM_RampSineHz_Ind].pid.Ki = PWM_RAMP_SPEED; hpwm1.hramp[PWM_RampSineHz_Ind].limMax = 656; hpwm1.hramp[PWM_RampSineHz_Ind].limMin = 0; hpwm1.hramp[PWM_RampSineHz_Ind].limMaxInt = 20; hpwm1.hramp[PWM_RampSineHz_Ind].limMinInt = -20; hpwm1.hramp[PWM_RampSineHz_Ind].SampleT = (1.0f/HZ_TIMER_CTRL); PWM_Ramp_UpdateParams(&hpwm1.hramp[PWM_RampSineHz_Ind], 1); // // DUTY RAMP PARAMS hpwm1.hramp[PWM_RampPWMDuty_Ind].pid.Ki = PWM_RAMP_SPEED; hpwm1.hramp[PWM_RampPWMDuty_Ind].limMax = 100; hpwm1.hramp[PWM_RampPWMDuty_Ind].limMin = 0; hpwm1.hramp[PWM_RampPWMDuty_Ind].limMaxInt = 20; hpwm1.hramp[PWM_RampPWMDuty_Ind].limMinInt = -20; hpwm1.hramp[PWM_RampPWMDuty_Ind].SampleT = (1.0f/HZ_TIMER_CTRL); PWM_Ramp_UpdateParams(&hpwm1.hramp[PWM_RampPWMDuty_Ind], 1); // PWM CONTROL PINS INIT GPIO_InitTypeDef GPIO_InitStruct = {0}; GPIO_Clock_Enable(GPIOC); GPIO_InitStruct.Pin = GPIO_PIN_0; GPIO_InitStruct.Mode = GPIO_MODE_OUTPUT_PP; GPIO_InitStruct.Pull = GPIO_PULLUP; GPIO_InitStruct.Speed = GPIO_SPEED_FREQ_LOW; HAL_GPIO_Init(GPIOC, &GPIO_InitStruct); GPIOC->ODR¦=1<<0;//подача 0 разрешает . Аппаратное разрешение и запрет не подачу шим . Cигнал идет на логический элемент & 74AC08d GPIO_InitStruct.Pin = GPIO_PIN_1; GPIO_InitStruct.Mode = GPIO_MODE_INPUT; GPIO_InitStruct.Pull = GPIO_PULLUP; GPIO_InitStruct.Speed = GPIO_SPEED_FREQ_LOW; HAL_GPIO_Init(GPIOC, &GPIO_InitStruct); //кнопка для вкл аппаратно шим 74AC08d //---------PWM TIMER1 INIT------------ // channels settings hpwm1.sConfigPWM.OCMode = TIM_OCMODE_PWM1; hpwm1.sConfigPWM.Pulse = 0; hpwm1.sConfigPWM.OCPolarity = TIM_OCPOLARITY_LOW; hpwm1.sConfigPWM.OCFastMode = TIM_OCFAST_DISABLE; // tim1 settings hpwm1.stim.htim.Instance = TIMER_PWM1_INSTANCE; // hpwm1.stim.htim.Init.CounterMode = TIM_COUNTERMODE_DOWN; hpwm1.stim.htim.Init.AutoReloadPreload = TIM_AUTORELOAD_PRELOAD_ENABLE; hpwm1.stim.sTimMode = TIM_IT_MODE; hpwm1.stim.sTimFreqHz = HZ_TIMER_PWM; hpwm1.stim.sTickBaseUS = PROJSET.TIM_PWM_TICKBASE; hpwm1.stim.sTimAHBFreqMHz = PROJSET.TIM_PWM_AHB_FREQ; hpwm1.GPIOx = TIMER_PWM1_GPIOx; hpwm1.GPIO_PIN_X1 = PROJSET.TIM_PWM1_GPIO_PIN_X1; hpwm1.GPIO_PIN_X2 = PROJSET.TIM_PWM1_GPIO_PIN_X2; hpwm1.sPWM_Channel1 = PROJSET.TIM_PWM1_TIM_CHANNEL1; hpwm1.sPWM_Channel2 = PROJSET.TIM_PWM1_TIM_CHANNEL2; hpwm1.hpwm2 = (void *)&hpwm2; hpwm1.hpwm3 = (void *)&hpwm3; TIM_Base_Init(&hpwm1.stim); TIM_Output_PWM_Init(&hpwm1.stim.htim, &hpwm1.sConfigPWM, hpwm1.sPWM_Channel1, hpwm1.GPIOx, hpwm1.GPIO_PIN_X1); TIM_Output_PWM_Init(&hpwm1.stim.htim, &hpwm1.sConfigPWM, hpwm1.sPWM_Channel2, hpwm1.GPIOx, hpwm1.GPIO_PIN_X2); // PWM SLAVES INIT hpwm2.hMasterPWM = &hpwm1; hpwm2.stim = hpwm1.stim; hpwm2.stim.htim.Instance = (TIM_TypeDef *)PROJSET.TIM_PWM2_INSTANCE; hpwm2.GPIOx = (GPIO_TypeDef *)PROJSET.TIM_PWM2_GPIOx; hpwm2.GPIO_PIN_X1 = PROJSET.TIM_PWM2_GPIO_PIN_X1; hpwm2.GPIO_PIN_X2 = PROJSET.TIM_PWM2_GPIO_PIN_X2; hpwm2.sPWM_Channel1 = PROJSET.TIM_PWM2_TIM_CHANNEL1; hpwm2.sPWM_Channel2 = PROJSET.TIM_PWM2_TIM_CHANNEL2; //hpwm2.Duty_Shift_Ratio = shiftPHASE1; hpwm2.Duty_Shift_Ratio = shiftPHASE1; hpwm3.hMasterPWM = &hpwm1; hpwm3.stim = hpwm1.stim; hpwm3.stim.htim.Instance = (TIM_TypeDef *)PROJSET.TIM_PWM3_INSTANCE; hpwm3.GPIOx = (GPIO_TypeDef *)PROJSET.TIM_PWM3_GPIOx; hpwm3.GPIO_PIN_X1 = PROJSET.TIM_PWM3_GPIO_PIN_X1; hpwm3.GPIO_PIN_X2 = PROJSET.TIM_PWM3_GPIO_PIN_X2; hpwm3.sPWM_Channel1 = PROJSET.TIM_PWM3_TIM_CHANNEL1; hpwm3.sPWM_Channel2 = PROJSET.TIM_PWM3_TIM_CHANNEL2; //hpwm3.Duty_Shift_Ratio = shiftPHASE2; hpwm3.Duty_Shift_Ratio = shiftPHASE2; PWM_SynchInit(); PWM_SlavePhase_Init(&hpwm2); PWM_SlavePhase_Init(&hpwm3); //----------TIMERS START------------- HAL_TIM_OC_Start(&hpwm1.stim.htim, TIM_CHANNEL_3); HAL_TIM_PWM_Start(&hpwm1.stim.htim, hpwm1.sPWM_Channel1); // PWM channel 1 HAL_TIM_PWM_Start(&hpwm1.stim.htim, hpwm1.sPWM_Channel2); // PWM channel 2 HAL_TIM_Base_Start_IT(&hpwm1.stim.htim); // timer for PWM } void PWM_SynchInit(void) { TIM_MasterConfigTypeDef sMasterConfig = {0}; TIM_OC_InitTypeDef sConfigOC = {0}; //--------MASTER SYNCH INIT---------- sMasterConfig.MasterOutputTrigger = TIM_TRGO_OC3REF; sMasterConfig.MasterSlaveMode = TIM_MASTERSLAVEMODE_DISABLE; sConfigOC.OCMode = TIM_OCMODE_ACTIVE; sConfigOC.Pulse = 0; sConfigOC.OCPolarity = TIM_OCPOLARITY_HIGH; HAL_TIMEx_MasterConfigSynchronization(&hpwm1.stim.htim, &sMasterConfig); HAL_TIM_OC_ConfigChannel(&hpwm1.stim.htim, &sConfigOC, TIM_CHANNEL_3); } /** * @brief PWM Handler. * @param hpwm - указатель на хендл ШИМ. * @details Управляет скважность ШИМ в режиме PWM_TABLE. * @note This called from TIM_PWM_Handler */ void PWM_Handler(PWM_HandleTypeDef *hpwm) { uint16_t rotate_ind_A; uint16_t rotate_ind_B; uint16_t rotate_ind_C; // rotate pwm rotate_ind_A = PWM_Get_Duty_Table_Ind(hpwm, hpwm->stim.sTimFreqHz); if (PWM_Get_Mode(hpwm, PWM_PHASE_MODE)) { rotate_ind_B = PWM_SlavePhase_Calc_TableInd(PWM_Set_pSlaveHandle(hpwm,hpwm2), rotate_ind_A); rotate_ind_C = PWM_SlavePhase_Calc_TableInd(PWM_Set_pSlaveHandle(hpwm,hpwm3), rotate_ind_A); } if((hpwm->PWM_Sine_Hz == 0 && (PWM_Get_Mode(&hpwm1,PWM_BRIDGE_MODE¦PWM_DC_MODE) != PWM_DC_MODE)) ¦¦ // if SINE HZ = 0 in SINE mode or BRIDGE mode (DC = 0, BRIDGE = 1) (hpwm->PWM_Duty == 0 && PWM_Get_Mode(&hpwm1,PWM_DC_MODE)) ) // if DUTY = 0 in DC MODE { //reset all channels PWM_Set_Compare1(hpwm, 0); // reset first channel PWM_Set_Compare2(hpwm, 0); // reset second channel PWM_Set_Compare1(PWM_Set_pSlaveHandle(hpwm,hpwm2), 0); // reset first channel PWM_Set_Compare2(PWM_Set_pSlaveHandle(hpwm,hpwm2), 0); // reset second channel PWM_Set_Compare1(PWM_Set_pSlaveHandle(hpwm,hpwm3), 0); // reset first channel PWM_Set_Compare2(PWM_Set_pSlaveHandle(hpwm,hpwm3), 0); // reset second channel return; } //--------BRIDGE MODE DISABLE-------- if(PWM_Get_Mode(&hpwm1,PWM_BRIDGE_MODE) == 0) { //-----------PWM DC MODE------------- if(PWM_Get_Mode(&hpwm1,PWM_DC_MODE)) PWM_SingleChannel_Mode(hpwm, rotate_ind_A, 0); //------------SINUS MODE------------- else PWM_SingleChannel_Mode(hpwm, rotate_ind_A, 1); } //---------BRIDGE MODE ENABLE-------- else { if(PWM_Get_Mode(&hpwm1,PWM_DC_MODE)) //-----------PWM DC MODE------------- PWM_DC_Bridge_Mode(hpwm, rotate_ind_A, rotate_ind_B, rotate_ind_C); //------------SINUS MODE------------- else PWM_Sine_Bridge_Mode(hpwm, rotate_ind_A, rotate_ind_B, rotate_ind_C); } //-----CHECK CHANNELS FOR ERRORS----- uint16_t min_duty = PWM_Calc_Min_Duty(hpwm); uint16_t max_duty = PWM_Calc_Max_Duty(hpwm); // IF FIRST CHANNEL IS ACRIVE if(PWM_Get_Compare1(hpwm) != 0) { // Duty shoud be bigger or equeal than min duration if (PWM_Get_Compare1(hpwm)<min_duty) PWM_Set_Compare1(hpwm, min_duty); // Duty shoud be less or equeal than ARR-min duration if (PWM_Get_Compare1(hpwm)>max_duty) PWM_Set_Compare1(hpwm, max_duty); } // IF SECOND CHANNEL IS ACRIVE else if(PWM_Get_Compare2(hpwm) != 0) { // Duty shoud be bigger or equeal than min duration if (PWM_Get_Compare2(hpwm)<min_duty) PWM_Set_Compare2(hpwm, min_duty); // Duty shoud be less or equeal than ARR if (PWM_Get_Compare2(hpwm)>max_duty) PWM_Set_Compare2(hpwm, max_duty); } // IF BOTH CHANNEL IS ACRIVE if((PWM_Get_Compare1(hpwm) != 0) && (PWM_Get_Compare2(hpwm) != 0)) { // Only one channel shoud be active so disable all PWM_Set_Compare1(hpwm, 0); PWM_Set_Compare2(hpwm, 0); } PWM_SlavePhase_Check_Channels(PWM_Set_pSlaveHandle(hpwm,hpwm2)); PWM_SlavePhase_Check_Channels(PWM_Set_pSlaveHandle(hpwm,hpwm3)); if(hpwm->PWM_DeadTime) { PWM_CreateDeadTime(hpwm, &DeadTimeCnt_Master, &ActiveChannelSHDW_Master); PWM_SlavePhase_CreateDeadTime(PWM_Set_pSlaveHandle(hpwm,hpwm2), &DeadTimeCnt_Slave2, &ActiveChannelSHDW_Slave2); PWM_SlavePhase_CreateDeadTime(PWM_Set_pSlaveHandle(hpwm,hpwm3), &DeadTimeCnt_Slave3, &ActiveChannelSHDW_Slave3); } } /** * @brief Form PWM on Single Channel (DC or Sine PWM). Slave PWM disabled. * @param hspwm - указатель на хендл слейв ШИМ. * @param rotate_ind_A - индекс таблицы для Мастер ШИМ. * @param SineOrDC - Флаг: скважность постоянная или меняющаяся по закону синусна. */ void PWM_SingleChannel_Mode(PWM_HandleTypeDef *hpwm, uint16_t rotate_ind_A, uint8_t SineOrDC) { PWM_SlaveHandleTypeDef *hspwm2 = PWM_Set_pSlaveHandle(hpwm,hpwm2); PWM_SlaveHandleTypeDef *hspwm3 = PWM_Set_pSlaveHandle(hpwm,hpwm3); // PWM_Set_Compare1(hpwm, 0); // reset first channel // PWM_Set_Compare2(hpwm, 0); // reset second channel // PWM_Set_Compare1(PWM_Set_pSlaveHandle(hpwm,hpwm2), 0); // reset first channel // PWM_Set_Compare2(PWM_Set_pSlaveHandle(hpwm,hpwm2), 0); // reset second channel // PWM_Set_Compare1(PWM_Set_pSlaveHandle(hpwm,hpwm3), 0); // reset first channel // PWM_Set_Compare2(PWM_Set_pSlaveHandle(hpwm,hpwm3), 0); // reset second channel switch(PWM_Get_Mode(hpwm, PWM_ACTIVECHANNEL_MODE)) { case PWM_ACTIVECHANNEL_A0: if(SineOrDC) PWM_Set_Duty_From_Table(hpwm, hpwm->sPWM_Channel1, rotate_ind_A); else PWM_Set_Duty_From_Value(hpwm, hpwm->sPWM_Channel1); // set first channel PWM_Set_Compare2(hpwm, 0); // reset second channel PWM_Set_Compare1(PWM_Set_pSlaveHandle(hpwm,hpwm2), 0); // reset first channel PWM_Set_Compare2(PWM_Set_pSlaveHandle(hpwm,hpwm2), 0); // reset second channel PWM_Set_Compare1(PWM_Set_pSlaveHandle(hpwm,hpwm3), 0); // reset first channel PWM_Set_Compare2(PWM_Set_pSlaveHandle(hpwm,hpwm3), 0); // reset second channel break; case PWM_ACTIVECHANNEL_A1: if(SineOrDC) PWM_Set_Duty_From_Table(hpwm, hpwm->sPWM_Channel2, rotate_ind_A); else PWM_Set_Duty_From_Value(hpwm, hpwm->sPWM_Channel2); // set second channel PWM_Set_Compare1(hpwm, 0); // reset first channel PWM_Set_Compare1(PWM_Set_pSlaveHandle(hpwm,hpwm2), 0); // reset first channel PWM_Set_Compare2(PWM_Set_pSlaveHandle(hpwm,hpwm2), 0); // reset second channel PWM_Set_Compare1(PWM_Set_pSlaveHandle(hpwm,hpwm3), 0); // reset first channel PWM_Set_Compare2(PWM_Set_pSlaveHandle(hpwm,hpwm3), 0); // reset second channel break; case PWM_ACTIVECHANNEL_B0: if(SineOrDC) PWM_Set_SlaveDuty_From_Table(hspwm2, hspwm2->sPWM_Channel1, rotate_ind_A); else PWM_Set_SlaveDuty_From_Value(hspwm2, hspwm2->sPWM_Channel1); // set first channel PWM_Set_Compare1(hpwm, 0); // reset first channel PWM_Set_Compare2(hpwm, 0); // reset second channel PWM_Set_Compare2(PWM_Set_pSlaveHandle(hpwm,hpwm2), 0); // reset second channel PWM_Set_Compare1(PWM_Set_pSlaveHandle(hpwm,hpwm3), 0); // reset first channel PWM_Set_Compare2(PWM_Set_pSlaveHandle(hpwm,hpwm3), 0); // reset second channel break; case PWM_ACTIVECHANNEL_B1: if(SineOrDC) PWM_Set_SlaveDuty_From_Table(hspwm2, hspwm2->sPWM_Channel2, rotate_ind_A); else PWM_Set_SlaveDuty_From_Value(hspwm2, hspwm2->sPWM_Channel2); // set second channel PWM_Set_Compare1(hpwm, 0); // reset first channel PWM_Set_Compare2(hpwm, 0); // reset second channel PWM_Set_Compare1(PWM_Set_pSlaveHandle(hpwm,hpwm2), 0); // reset first channel PWM_Set_Compare1(PWM_Set_pSlaveHandle(hpwm,hpwm3), 0); // reset first channel PWM_Set_Compare2(PWM_Set_pSlaveHandle(hpwm,hpwm3), 0); // reset second channel break; case PWM_ACTIVECHANNEL_C0: if(SineOrDC) PWM_Set_SlaveDuty_From_Table(hspwm3, hspwm3->sPWM_Channel1, rotate_ind_A); else PWM_Set_SlaveDuty_From_Value(hspwm3, hspwm3->sPWM_Channel1); // set first channel PWM_Set_Compare1(hpwm, 0); // reset first channel PWM_Set_Compare2(hpwm, 0); // reset second channel PWM_Set_Compare1(PWM_Set_pSlaveHandle(hpwm,hpwm2), 0); // reset first channel PWM_Set_Compare2(PWM_Set_pSlaveHandle(hpwm,hpwm2), 0); // reset second channel PWM_Set_Compare2(PWM_Set_pSlaveHandle(hpwm,hpwm3), 0); // reset second channel break; case PWM_ACTIVECHANNEL_C1: if(SineOrDC) PWM_Set_SlaveDuty_From_Table(hspwm3, hspwm3->sPWM_Channel2, rotate_ind_A); else PWM_Set_SlaveDuty_From_Value(hspwm3, hspwm3->sPWM_Channel2); // set second channel PWM_Set_Compare1(hpwm, 0); // reset first channel PWM_Set_Compare2(hpwm, 0); // reset second channel PWM_Set_Compare1(PWM_Set_pSlaveHandle(hpwm,hpwm2), 0); // reset first channel PWM_Set_Compare2(PWM_Set_pSlaveHandle(hpwm,hpwm2), 0); // reset second channel PWM_Set_Compare1(PWM_Set_pSlaveHandle(hpwm,hpwm3), 0); // reset first channel break; default: PWM_Set_Compare1(hpwm, 0); // reset first channel PWM_Set_Compare2(hpwm, 0); // reset second channel PWM_Set_Compare1(PWM_Set_pSlaveHandle(hpwm,hpwm2), 0); // reset first channel PWM_Set_Compare2(PWM_Set_pSlaveHandle(hpwm,hpwm2), 0); // reset second channel PWM_Set_Compare1(PWM_Set_pSlaveHandle(hpwm,hpwm3), 0); // reset first channel PWM_Set_Compare2(PWM_Set_pSlaveHandle(hpwm,hpwm3), 0); // reset second channel break; } } /** * @brief Form Bridge Sine PWM. Forming Slave PWM included. * @param hspwm - указатель на хендл слейв ШИМ. * @param rotate_ind_A - индекс таблицы для Мастер ШИМ. * @param rotate_ind_B - индекс таблицы для Слейв ШИМ 2. * @param rotate_ind_C - индекс таблицы для Слейв ШИМ 3. * @note Индекс для свейл ШИМ расчитывается внутри функции. */ void PWM_Sine_Bridge_Mode(PWM_HandleTypeDef *hpwm, uint16_t rotate_ind_A, uint16_t rotate_ind_B, uint16_t rotate_ind_C) { if(hpwm->PWM_MaxPulseDur - hpwm->PWM_MinPulseDur > 2) { //----------MASTER PWM----------- // SET PHASE 1 (A) int Duty = PWM_Scaled_Element(hpwm, PWM_Get_Table_Element_Signed(hpwm, rotate_ind_A)); // если это первая полуволна if(Duty > 0) { PWM_Set_Compare1(hpwm, Duty+PWM_Calc_Min_Duty(hpwm)); // set first channel PWM_Set_Compare2(hpwm, 0); // reset second channel } // если это вторая полуволна else { PWM_Set_Compare1(hpwm, 0); // reset first channel PWM_Set_Compare2(hpwm, (-Duty)+PWM_Calc_Min_Duty(hpwm)); // set second channel } //----------SLAVE PWMs----------- if (PWM_Get_Mode(hpwm, PWM_PHASE_MODE)) { // SET PHASE 2 (B) PWM_SlaveHandleTypeDef *hspwm = PWM_Set_pSlaveHandle(hpwm,hpwm2); Duty = PWM_Scaled_Element(hpwm, PWM_Get_Table_Element_Signed(hpwm, rotate_ind_B)); // если это первая полуволна if(Duty > 0) { PWM_Set_Compare1(hspwm, Duty+PWM_Calc_Min_Duty(hspwm->hMasterPWM)); // set first channel PWM_Set_Compare2(hspwm, 0); // reset second channel } // если это вторая полуволна else { PWM_Set_Compare1(hspwm, 0); // reset first channel PWM_Set_Compare2(hspwm, (-Duty)+PWM_Calc_Min_Duty(hspwm->hMasterPWM)); // set second channel } // SET PHASE 3 (C) hspwm = PWM_Set_pSlaveHandle(hpwm,hpwm3); Duty = PWM_Scaled_Element(hpwm, PWM_Get_Table_Element_Signed(hpwm, rotate_ind_C)); // если это первая полуволна if(Duty > 0) { PWM_Set_Compare1(hspwm, Duty+PWM_Calc_Min_Duty(hspwm->hMasterPWM)); // set first channel PWM_Set_Compare2(hspwm, 0); // reset second channel } // если это вторая полуволна else { PWM_Set_Compare1(hspwm, 0); // reset first channel PWM_Set_Compare2(hspwm, (-Duty)+PWM_Calc_Min_Duty(hspwm->hMasterPWM)); // set second channel } } else { PWM_Set_Compare1(PWM_Set_pSlaveHandle(hpwm,hpwm2), 0); // reset first channel PWM_Set_Compare2(PWM_Set_pSlaveHandle(hpwm,hpwm2), 0); // reset second channel PWM_Set_Compare1(PWM_Set_pSlaveHandle(hpwm,hpwm3), 0); // reset first channel PWM_Set_Compare2(PWM_Set_pSlaveHandle(hpwm,hpwm3), 0); // reset second channel } } else { PWM_Set_Compare1(hpwm, 0); // reset first channel PWM_Set_Compare2(hpwm, 0); // reset second channel PWM_Set_Compare1(PWM_Set_pSlaveHandle(hpwm,hpwm2), 0); // reset first channel PWM_Set_Compare2(PWM_Set_pSlaveHandle(hpwm,hpwm2), 0); // reset second channel PWM_Set_Compare1(PWM_Set_pSlaveHandle(hpwm,hpwm3), 0); // reset first channel PWM_Set_Compare2(PWM_Set_pSlaveHandle(hpwm,hpwm3), 0); // reset second channel } } /** * @brief Form Bridge DC PWM. Forming Slave PWM included. * @param hspwm - указатель на хендл слейв ШИМ. * @param rotate_ind_A - индекс таблицы для Мастер ШИМ. * @param rotate_ind_B - индекс таблицы для Слейв ШИМ 2. * @param rotate_ind_C - индекс таблицы для Слейв ШИМ 3. * @note Индекс для свейл ШИМ расчитывается внутри функции. */ void PWM_DC_Bridge_Mode(PWM_HandleTypeDef *hpwm, uint16_t rotate_ind_A, uint16_t rotate_ind_B, uint16_t rotate_ind_C) { //----------MASTER PWM----------- // SET PHASE 1 (A) int Duty = PWM_Get_Table_Element_Signed(hpwm, rotate_ind_A); // если это первая полуволна if(Duty > 0) { PWM_Set_Duty_From_Value(hpwm, hpwm->sPWM_Channel1); // set first channel PWM_Set_Compare2(hpwm, 0); // reset second channel } // если это вторая полуволна else { PWM_Set_Compare1(hpwm, 0); // reset first channel PWM_Set_Duty_From_Value(hpwm, hpwm->sPWM_Channel2); // set second channel } //----------SLAVE PWMs----------- if (PWM_Get_Mode(hpwm, PWM_PHASE_MODE)) { // SET PHASE 2 (B) PWM_SlaveHandleTypeDef *hspwm = PWM_Set_pSlaveHandle(hpwm,hpwm2); Duty = PWM_Get_Table_Element_Signed(hpwm, rotate_ind_B); // если это первая полуволна if(Duty > 0) { PWM_Set_SlaveDuty_From_Value(hspwm, hspwm->sPWM_Channel1); // set first channel PWM_Set_Compare2(hspwm, 0); // reset second channel } // если это вторая полуволна else { PWM_Set_Compare1(hspwm, 0); // reset first channel PWM_Set_SlaveDuty_From_Value(hspwm, hspwm->sPWM_Channel2); // set second channel } // SET PHASE 3 (C) hspwm = PWM_Set_pSlaveHandle(hpwm,hpwm3); Duty = PWM_Get_Table_Element_Signed(hpwm, rotate_ind_C); // если это первая полуволна if(Duty > 0) { PWM_Set_SlaveDuty_From_Value(hspwm, hspwm->sPWM_Channel1); // set first channel PWM_Set_Compare2(hspwm, 0); // reset second channel } // если это вторая полуволна else { PWM_Set_Compare1(hspwm, 0); // reset first channel PWM_Set_SlaveDuty_From_Value(hspwm, hspwm->sPWM_Channel2); // set second channel } } else { PWM_Set_Compare1(PWM_Set_pSlaveHandle(hpwm,hpwm2), 0); // reset first channel PWM_Set_Compare2(PWM_Set_pSlaveHandle(hpwm,hpwm2), 0); // reset second channel PWM_Set_Compare1(PWM_Set_pSlaveHandle(hpwm,hpwm3), 0); // reset first channel PWM_Set_Compare2(PWM_Set_pSlaveHandle(hpwm,hpwm3), 0); // reset second channel } } /** * @brief Update PWM parameters. * @param hpwm - указатель на хендл ШИМ. * @details Проверка надо ли обновлять параметры ШИМ, и если надо - обновляет их. * @note This called from TIM_CTRL_Handler */ void PWM_Update_Params(PWM_HandleTypeDef *hpwm) { unsigned UpdateModeParams = 0; unsigned UpdateLog = 0; static float PWM_Sine_setpoint; static float PWM_Duty_setpoint; // READ PWM_SINE_HZ if(PWM_Sine_setpoint != int_to_percent(pwm_ctrl[R_PWM_CTRL_PWM_SINE_HZ])) { PWM_Sine_setpoint = int_to_percent(pwm_ctrl[R_PWM_CTRL_PWM_SINE_HZ]); if((PWM_Sine_setpoint == 0) ¦¦ // if zero - force reset pwm value (hpwm->PWM_Sine_Hz != hpwm->PWM_Sine_Hz ) ) // or if PWM_Sine_Hz isnt in range { PWM_Ramp_Reset(&hpwm->hramp[PWM_RampSineHz_Ind]); } // update logs params UpdateLog = 1; } // READ DUTY_BRIDGE if(PWM_Duty_setpoint != pwm_ctrl[R_PWM_CTRL_DUTY_BRIDGE]) { PWM_Duty_setpoint = int_to_percent(pwm_ctrl[R_PWM_CTRL_DUTY_BRIDGE]); if((PWM_Duty_setpoint == 0) ¦¦ // if zero - force reset pwm value (hpwm->PWM_Duty != hpwm->PWM_Duty ) ) // or if PWM_Duty isnt in range { PWM_Ramp_Reset(&hpwm->hramp[PWM_RampPWMDuty_Ind]); } // update mode params UpdateModeParams = 1; // update logs params UpdateLog = 1; } // UPDATE RAMP SINE HZ PWM_Ramp_ControlValue(&hpwm->hramp[PWM_RampSineHz_Ind], &hpwm->PWM_Sine_Hz, PWM_Sine_setpoint); // UPDATE RAMP PWM_Ramp_ControlValue(&hpwm->hramp[PWM_RampPWMDuty_Ind], &hpwm->PWM_Duty, PWM_Duty_setpoint); // READ PWM_ACTIVECHANNEL_MODE uint16_t activechannesls = (MB_Read_Coil_Local(&coils_regs[0], COIL_PWM_ACTIVECHANNEL))¦ (MB_Read_Coil_Local(&coils_regs[0], COIL_PWM_ACTIVECHANNEL+1) << 1)¦ (MB_Read_Coil_Local(&coils_regs[0], COIL_PWM_ACTIVECHANNEL+2) << 2); if(PWM_Get_Mode(hpwm, PWM_ACTIVECHANNEL_MODE) != (activechannesls << PWM_ACTIVECHANNEL_MODE_Pos)) { uint16_t tmpmode = hpwm->sPWM_Mode&(~PWM_ACTIVECHANNEL_MODE); tmpmode ¦= activechannesls << PWM_ACTIVECHANNEL_MODE_Pos; hpwm->sPWM_Mode = tmpmode; // update mode params UpdateModeParams = 1; // update logs params UpdateLog = 1; } // READ PWM_DC_MODE if(PWM_Get_Mode(hpwm, PWM_DC_MODE) != (MB_Read_Coil_Local(&coils_regs[0], COIL_PWM_DC_MODE) << PWM_DC_MODE_Pos)) { if(MB_Read_Coil_Local(&coils_regs[0], COIL_PWM_DC_MODE)) { hpwm->sPWM_Mode ¦= PWM_DC_MODE; } else { hpwm->sPWM_Mode &= ~PWM_DC_MODE; } // update mode params UpdateModeParams = 1; // update logs params UpdateLog = 1; } // READ PWM_BRIDGE_MODE if(PWM_Get_Mode(hpwm, PWM_BRIDGE_MODE) != (MB_Read_Coil_Local(&coils_regs[0], COIL_PWM_BRIDGE_MODE) << PWM_BRIDGE_MODE_Pos)) { if(MB_Read_Coil_Local(&coils_regs[0], COIL_PWM_BRIDGE_MODE)) { hpwm->sPWM_Mode ¦= PWM_BRIDGE_MODE; } else { hpwm->sPWM_Mode &= ~PWM_BRIDGE_MODE; } // update mode params UpdateModeParams = 1; // update logs params UpdateLog = 1; } // READ PWM_PHASE_MODE if(PWM_Get_Mode(hpwm, PWM_PHASE_MODE) != (MB_Read_Coil_Local(&coils_regs[0], COIL_PWM_PHASE_MODE) << PWM_PHASE_MODE_Pos)) { if(MB_Read_Coil_Local(&coils_regs[0], COIL_PWM_PHASE_MODE)) { hpwm->sPWM_Mode ¦= PWM_PHASE_MODE; } else { hpwm->sPWM_Mode &= ~PWM_PHASE_MODE; } // update mode params UpdateModeParams = 1; // update logs params UpdateLog = 1; } // READ PWM_POLARITY uint16_t polarity_temp = MB_Read_Coil_Local(&coils_regs[0], COIL_PWM_POLARITY); if(((hpwm->stim.htim.Instance->CCER&TIM_CCER_CC1P) ¦¦ (hpwm->stim.htim.Instance->CCER&TIM_CCER_CC3P)) != (polarity_temp)) { __HAL_TIM_SET_CAPTUREPOLARITY(&hpwm->stim.htim, hpwm->sPWM_Channel1, polarity_temp<<1); __HAL_TIM_SET_CAPTUREPOLARITY(&hpwm->stim.htim, hpwm->sPWM_Channel2, polarity_temp<<1); PWM_SlaveHandleTypeDef *shpwm = PWM_Set_pSlaveHandle(hpwm,hpwm2); __HAL_TIM_SET_CAPTUREPOLARITY(&shpwm->stim.htim, shpwm->sPWM_Channel1, polarity_temp<<1); __HAL_TIM_SET_CAPTUREPOLARITY(&shpwm->stim.htim, shpwm->sPWM_Channel2, polarity_temp<<1); shpwm = PWM_Set_pSlaveHandle(hpwm,hpwm3); __HAL_TIM_SET_CAPTUREPOLARITY(&shpwm->stim.htim, shpwm->sPWM_Channel1, polarity_temp<<1); __HAL_TIM_SET_CAPTUREPOLARITY(&shpwm->stim.htim, shpwm->sPWM_Channel2, polarity_temp<<1); } // READ TABLE_SIZE if(hpwm->Duty_Table_Size != pwm_ctrl[R_PWM_CTRL_SIN_TABLE_SIZE]) { hpwm->Duty_Table_Size = PWM_Fill_Sine_Table(&hpwm1, pwm_ctrl[R_PWM_CTRL_SIN_TABLE_SIZE]); pwm_ctrl[R_PWM_CTRL_SIN_TABLE_SIZE] = hpwm->Duty_Table_Size; } // READ MIN PULSE DURATION if(hpwm->PWM_MinPulseDur != pwm_ctrl[R_PWM_CTRL_MIN_PULSE_DUR]) { hpwm->PWM_MinPulseDur = pwm_ctrl[R_PWM_CTRL_MIN_PULSE_DUR]; // update mode params UpdateModeParams = 1; // update logs params UpdateLog = 1; } // READ MAX PULSE DURATION if((hpwm->PWM_MaxPulseDur != pwm_ctrl[R_PWM_CTRL_MAX_PULSE_DUR]) && (pwm_ctrl[R_PWM_CTRL_MAX_PULSE_DUR] <= PWM_Get_Autoreload(hpwm)) && (pwm_ctrl[R_PWM_CTRL_MAX_PULSE_DUR] != 0)) { hpwm->PWM_MaxPulseDur = pwm_ctrl[R_PWM_CTRL_MAX_PULSE_DUR]; // update mode params UpdateModeParams = 1; // update logs params UpdateLog = 1; } else if (((hpwm->PWM_MaxPulseDur != PWM_Get_Autoreload(hpwm)) && (pwm_ctrl[R_PWM_CTRL_MAX_PULSE_DUR] == 0)) ¦¦ ((hpwm->PWM_MaxPulseDur != PWM_Get_Autoreload(hpwm)) && (pwm_ctrl[R_PWM_CTRL_MAX_PULSE_DUR] > PWM_Get_Autoreload(hpwm)))) { hpwm->PWM_MaxPulseDur = PWM_Get_Autoreload(hpwm); // update mode params UpdateModeParams = 1; } // READ DEAD TIME if(hpwm->PWM_DeadTime != pwm_ctrl[R_PWM_CTRL_DEAD_TIME]) { hpwm->PWM_DeadTime = pwm_ctrl[R_PWM_CTRL_DEAD_TIME]; } // UPDATE PWM PARAMS if(UpdateModeParams) { // UPDATE DUTY TABLE SCALE PWM_Update_DutyTableScale(hpwm); // update logs params UpdateLog = 1; } // UPDATE LOG PARAMS if(UpdateLog) { // set logs params Set_Log_Params(); } } /** * @brief PID for ramp. * @param hramp - указатель на хендл PID-регулятора. * @param PID_Output - указатель на переменную для выхода регулятора. * @param PID_Input - задание на ПИД-регулятор. * @details ПИД-регулятор, который управляет скважностью ШИМ и не дает её изменяться резко. */ void PWM_Ramp_ControlValue(PWM_RampHandleTypeDef *hramp, float *PID_Output, float PID_Input) { float out; float err = PID_Input-*PID_Output; /* Limiting the accelerate */ if (err > hramp->limMaxInt) err = hramp->limMaxInt; else if (err < hramp->limMinInt) err = hramp->limMinInt; out = arm_pid_f32(&hramp->pid, err); /* Limiting the accelerate */ if (out > hramp->limMax) out = hramp->limMax; else if (out < hramp->limMin) out = hramp->limMin; /* Apply the PID-regulating */ if(PID_Input != 0) { PWM_Ramp_UpdateParams(hramp, 0); *PID_Output = out; } else // if input = 0 - disable pid and set 0 { PWM_Ramp_Reset(hramp); *PID_Output = 0; } } /** * @brief Initialize PID params for ramp. * @param hramp - указатель на хендл PID-регулятора. * @param flagReset - флаг для сброса ПИД-регулятора (сброс регулирования, не параметров). * @details Обновляет параметры A0, A1, A2 для регилуривания @ref PID Motor Control */ void PWM_Ramp_UpdateParams(PWM_RampHandleTypeDef *hramp, int32_t flagReset) { // hramp->pid.Ki arm_pid_init_f32(&hramp->pid, flagReset); } /** * @brief Reset PID for ramp. * @param hramp - указатель на хендл PID-регулятора. * @details Сбрасывает переменные ПИД-регулятора, чтобы потом его запустить "с чистого листа". */ void PWM_Ramp_Reset(PWM_RampHandleTypeDef *hramp) { arm_pid_reset_f32(&hramp->pid); } /** * @brief reInitialization of PWM TIM. * @param hpwm - указатель на хендл ШИМ. * @details Перенастраивает таймер согласно принятным настройкам в pwm_ctrl. */ void PWM_Sine_ReInit(PWM_HandleTypeDef *hpwm) { Trace_PWM_reInit_Enter(); TIM_Base_MspDeInit(&hpwm->stim.htim); hpwm1.stim.sTickBaseUS = TIMER_PWM_TICKBASE; hpwm1.sConfigPWM.OCPolarity = MB_Read_Coil_Local(&coils_regs[0], COIL_PWM_POLARITY) << TIM_CCER_CC1P_Pos; TIM_Base_Init(&hpwm->stim); TIM_Output_PWM_Init(&hpwm->stim.htim, &hpwm->sConfigPWM, hpwm->sPWM_Channel1, hpwm->GPIOx, hpwm->GPIO_PIN_X1); TIM_Output_PWM_Init(&hpwm->stim.htim, &hpwm->sConfigPWM, hpwm->sPWM_Channel2, hpwm->GPIOx, hpwm->GPIO_PIN_X2); PWM_SynchInit(); PWM_SlavePhase_reInit(&hpwm2); PWM_SlavePhase_reInit(&hpwm3); PWM_Update_DutyTableScale(hpwm); //----------TIMERS START------------- HAL_TIM_OC_Start(&hpwm1.stim.htim, TIM_CHANNEL_3); HAL_TIM_PWM_Start(&hpwm1.stim.htim, hpwm->sPWM_Channel1); // PWM channel 1 HAL_TIM_PWM_Start(&hpwm1.stim.htim, hpwm->sPWM_Channel2); // PWM channel 2 HAL_TIM_Base_Start_IT(&hpwm1.stim.htim); // timer for PWM Trace_PWM_reInit_Exit(); } /** * @brief Getting ind for Duty Table. * @param hpwm - указатель на хендл ШИМ. * @param FreqTIM - частота таймера ШИМ. * @details Рассчитывает индекс для таблицы скважностей. * PWM_Sine_Hz в hpwm - частота с которой эта таблица должна выводиться на ШИМ */ uint32_t PWM_Get_Duty_Table_Ind(PWM_HandleTypeDef *hpwm, float FreqTIM) { float sine_ind_step; // calc ind for sin table if(hpwm->PWM_Sine_Hz != 0) // if there some frequency { sine_ind_step = hpwm->Duty_Table_Size/(FreqTIM/hpwm->PWM_Sine_Hz); hpwm->Duty_Table_Ind += sine_ind_step; } else return hpwm->Duty_Table_Size; // overflow check if(hpwm->Duty_Table_Ind >= hpwm->Duty_Table_Size) hpwm->Duty_Table_Ind -= hpwm->Duty_Table_Size; if(hpwm->Duty_Table_Ind >= hpwm->Duty_Table_Size) hpwm->Duty_Table_Ind = 0; // if its too big (e.g. inf) if((hpwm->Duty_Table_Ind >= 0xFFFF) ¦¦ (hpwm->Duty_Table_Ind != hpwm->Duty_Table_Ind)) // in nan case hpwm->Duty_Table_Ind = 0; return hpwm->Duty_Table_Ind; } /** * @brief Create Dead Time when switches channels. * @param hpwm - указатель на хендл ШИМ. * @param LocalDeadTimeCnt - указатель на переменную для отсчитывания дедтайма. * @param LocalActiveChannel - указатель на переменную для отслеживания смены канала. */ void PWM_CreateDeadTime(PWM_HandleTypeDef *hpwm, float *LocalDeadTimeCnt, unsigned *LocalActiveChannel) { // get current active channel hpwm->fActiveChannel = (PWM_Get_Compare2(hpwm) != 0); // if channel two is active - write 1, otherwise - 0 // when channels are swithed and no dead time currently active if(*LocalActiveChannel != hpwm->fActiveChannel) { // update active channel *LocalActiveChannel = hpwm->fActiveChannel; // set deadtime *LocalDeadTimeCnt = hpwm->PWM_DeadTime; Trace_PWM_DeadTime_Enter(); } // decrement dead time *LocalDeadTimeCnt -= (PWM_Get_Autoreload(hpwm)+1)*hpwm->stim.sTickBaseUS; if(*LocalDeadTimeCnt > 0) // if dead time is still active { // reset all channels // reset channels PWM_Set_Compare1(hpwm, 0); PWM_Set_Compare2(hpwm, 0); } else // if dead time is done { // set it to zero *LocalDeadTimeCnt = 0; Trace_PWM_DeadTime_Exit(); } } /** * @brief Filling table with one period of sinus values. * @param hpwm - указатель на хендл ШИМ. * @param table_size - размер таблицы. * @details Формирует таблицу синусов размером table_size. */ uint32_t PWM_Fill_Sine_Table(PWM_HandleTypeDef *hpwm, uint32_t table_size) { if((hpwm == NULL) ¦¦ (hpwm->pDuty_Table_Origin == NULL) ¦¦ (table_size == 0)) { return 0; } if (table_size > SIN_TABLE_SIZE_MAX) table_size = SIN_TABLE_SIZE_MAX; hpwm->Duty_Table_Size = table_size; float pi_step = 2*M_PI/(hpwm->Duty_Table_Size); float pi_val = 0; float sin_koef = 0; uint32_t sin_val = 0; // fill table with sinus for(int i = 0; i < hpwm->Duty_Table_Size; i++) { // rotate pi pi_val += pi_step; // calc sin value sin_koef = (float)0xFFFF; sin_val = (sin(pi_val)+1)*sin_koef/2; sin_table[i] = sin_val; } // fill rest of table with zeros for(int i = hpwm->Duty_Table_Size; i < SIN_TABLE_SIZE_MAX; i++) sin_table[i] = 0; // if second channel is enabled PWM_Update_DutyTableScale(hpwm); return hpwm->Duty_Table_Size; } /** * @brief Calc and update new Duty Table Scale. * @param hpwm - указатель на хендл ШИМ. * @details Используется, когда изменяется значение регистра ARR. */ void PWM_Update_DutyTableScale(PWM_HandleTypeDef *hpwm) { // UPDATE DUTY TABLE SCALE if(PWM_Get_Mode(hpwm, PWM_BRIDGE_MODE)) // if second channel is enabled { hpwm->Duty_Table_Scale = PWM_Calc_Duty_Scale(&hpwm1, 0x8000); } else { hpwm->Duty_Table_Scale = PWM_Calc_Duty_Scale(&hpwm1, 0xFFFF); } // for case if min pulse dur is too big and scale is negative if (hpwm->Duty_Table_Scale < 0) hpwm->Duty_Table_Scale = 1; } //------------------------------------------------------------------- //-----------------------THREEPHASE FUNCTIONS------------------------ /** * @brief Initialization of Slave PWM TIM. * @param hspwm - указатель на хендл слейв ШИМ. * @details Вызывает функции инициализации и включения слейв ШИМ. */ void PWM_SlavePhase_Init(PWM_SlaveHandleTypeDef *hspwm) { TIM_Base_Init(&hspwm->stim); TIM_Output_PWM_Init(&hspwm->stim.htim, &hspwm->hMasterPWM->sConfigPWM, hspwm->sPWM_Channel1, hspwm->GPIOx, hspwm->GPIO_PIN_X1); TIM_Output_PWM_Init(&hspwm->stim.htim, &hspwm->hMasterPWM->sConfigPWM, hspwm->sPWM_Channel2, hspwm->GPIOx, hspwm->GPIO_PIN_X2); TIM_MasterConfigTypeDef sMasterConfig = {0}; TIM_SlaveConfigTypeDef sSlaveConfig = {0}; //--------SLAVE SYNCH INIT---------- sSlaveConfig.SlaveMode = TIM_SLAVEMODE_TRIGGER; sSlaveConfig.InputTrigger = TIM_TS_ITR3; sMasterConfig.MasterOutputTrigger = TIM_TRGO_OC2REF; sMasterConfig.MasterSlaveMode = TIM_MASTERSLAVEMODE_DISABLE; HAL_TIM_SlaveConfigSynchro(&hpwm2.stim.htim, &sSlaveConfig); HAL_TIMEx_MasterConfigSynchronization(&hpwm2.stim.htim, &sMasterConfig); HAL_TIM_SlaveConfigSynchro(&hpwm3.stim.htim, &sSlaveConfig); HAL_TIMEx_MasterConfigSynchronization(&hpwm3.stim.htim, &sMasterConfig); //----------TIMERS START------------- HAL_TIM_Base_Start(&hspwm->stim.htim); HAL_TIM_PWM_Start(&hspwm->stim.htim, hspwm->sPWM_Channel1); // PWM channel 1 HAL_TIM_PWM_Start(&hspwm->stim.htim, hspwm->sPWM_Channel2); // PWM channel 2 if(PWM_Get_Mode(hspwm->hMasterPWM, PWM_PHASE_MODE) == 0) // if three phase disabled { PWM_Set_Compare1(hspwm, 0); // reset first channel PWM_Set_Compare2(hspwm, 0); // reset second channel } } /** * @brief reInitialization of Slave PWM TIM. * @param hspwm - указатель на хендл слейв ШИМ. */ void PWM_SlavePhase_reInit(PWM_SlaveHandleTypeDef *hspwm) { PWM_Slave_CopyTimSetting(hspwm, sTimFreqHz); PWM_Slave_CopyTimSetting(hspwm, sTickBaseUS); PWM_Slave_CopyTimSetting(hspwm, sTickBasePrescaler); TIM_Base_MspDeInit(&hspwm->stim.htim); PWM_SlavePhase_Init(hspwm); } /** * @brief Set Duty from table on Slave PWM at one channel by sin_ind of the Master PWM. * @param hspwm - указатель на хендл слейв ШИМ. * @param sin_ind - индекс таблицы для Мастер ШИМ. * @note Индекс для свейл ШИМ расчитывается в самой функции. */ uint16_t PWM_SlavePhase_Calc_TableInd(PWM_SlaveHandleTypeDef *hspwm, uint16_t rotate_ind_Phase) { // if three phase enables if(hspwm->Duty_Shift_Ratio > 0) rotate_ind_Phase += hspwm->hMasterPWM->Duty_Table_Size*hspwm->Duty_Shift_Ratio; else rotate_ind_Phase += hspwm->hMasterPWM->Duty_Table_Size*(1+hspwm->Duty_Shift_Ratio); // overflow check if(rotate_ind_Phase > hspwm->hMasterPWM->Duty_Table_Size) rotate_ind_Phase -= hspwm->hMasterPWM->Duty_Table_Size; if(rotate_ind_Phase > hspwm->hMasterPWM->Duty_Table_Size) rotate_ind_Phase = 0;; return rotate_ind_Phase; // // PWM_Set_SlaveDuty_From_Table(hspwm, sin_ind); // set first channel } /** * @brief Check is all Slave channels works properly. * @param hspwm - указатель на хендл слейв ШИМ. * @details Проверка работает ли только один из каналов, и проверка чтобы CCRx <= ARR * @details В мастере проверка происходит напрямую в PWM_Handler. */ void PWM_SlavePhase_Check_Channels(PWM_SlaveHandleTypeDef *hspwm) { uint16_t min_duty = PWM_Calc_Min_Duty(hspwm->hMasterPWM); uint16_t max_duty = PWM_Calc_Max_Duty(hspwm->hMasterPWM); // IF FIRST CHANNEL IS ACRIVE if(PWM_Get_Compare1(hspwm) != 0) { // Duty shoud be bigger or equeal than min duration if (PWM_Get_Compare1(hspwm)<min_duty) PWM_Set_Compare1(hspwm, min_duty); // Duty shoud be less or equeal than ARR-min duration if (PWM_Get_Compare1(hspwm)>max_duty) PWM_Set_Compare1(hspwm, max_duty); } // IF SECOND CHANNEL IS ACRIVE else if(PWM_Get_Compare2(hspwm) != 0) // Duty shoud be bigger or equeal than min duration if (PWM_Get_Compare2(hspwm)<min_duty) PWM_Set_Compare2(hspwm, min_duty); // Duty shoud be less or equeal than ARR if (PWM_Get_Compare2(hspwm)>max_duty) PWM_Set_Compare2(hspwm, max_duty); // IF BOTH CHANNEL IS ACRIVE if((PWM_Get_Compare1(hspwm) != 0) && (PWM_Get_Compare2(hspwm) != 0)) { // Only one channel shoud be active so disable all PWM_Set_Compare1(hspwm, 0); PWM_Set_Compare2(hspwm, 0); } } /** * @brief Create Dead Time for Slave PWM when switches channels. * @param hspwm - указатель на хендл слейв ШИМ. * @param LocalDeadTimeCnt - указатель на переменную для отсчитывания дедтайма. * @param LocalActiveChannel - указатель на переменную для отслеживания смены канала. * @note Аналог функции PWM_CreateDeadTime но для слейв ШИМов. */ void PWM_SlavePhase_CreateDeadTime(PWM_SlaveHandleTypeDef *hspwm, float *LocalDeadTimeCnt, unsigned *LocalActiveChannel) { // get current active channel hspwm->fActiveChannel = (PWM_Get_Compare2(hspwm) != 0); // if channel two is active - write 1, otherwise - 0 // when channels are swithed and no dead time currently active if(*LocalActiveChannel != hspwm->fActiveChannel) { // update active channel *LocalActiveChannel = hspwm->fActiveChannel; // set deadtime *LocalDeadTimeCnt = hspwm->hMasterPWM->PWM_DeadTime; Trace_PWM_DeadTime_Enter(); } // decrement dead time *LocalDeadTimeCnt -= (PWM_Get_Autoreload(hspwm)+1)*hspwm->hMasterPWM->stim.sTickBaseUS; if(*LocalDeadTimeCnt > 0) // if dead time is still active { // reset all channels // reset channels PWM_Set_Compare1(hspwm, 0); PWM_Set_Compare2(hspwm, 0); } else // if dead time is done { // set it to zero *LocalDeadTimeCnt = 0; Trace_PWM_DeadTime_Exit(); } } //------------------------------------------------------------------- //------------------------HANDLERS FUNCTIONS------------------------- //---------------PWM TIMER----------------- #if (PWM_MASTER_TIM_NUMB == 1) ¦¦ (PWM_MASTER_TIM_NUMB == 10) // choose handler for TIM void TIM1_UP_TIM10_IRQHandler(void) #elif (PWM_MASTER_TIM_NUMB == 2) void TIM2_IRQHandler(void) #elif (PWM_MASTER_TIM_NUMB == 3) void TIM3_IRQHandler(void) #elif (PWM_MASTER_TIM_NUMB == 4) void TIM4_IRQHandler(void) #elif (PWM_MASTER_TIM_NUMB == 5) void TIM5_IRQHandler(void) #elif (PWM_MASTER_TIM_NUMB == 6) void TIM6_DAC_IRQHandler(void) #elif (PWM_MASTER_TIM_NUMB == 7) void TIM7_IRQHandler(void) #elif (PWM_MASTER_TIM_NUMB == 8) ¦¦ (PWM_MASTER_TIM_NUMB == 13) void TIM8_UP_TIM13_IRQHandler(void) #elif (PWM_MASTER_TIM_NUMB == 1) ¦¦ (PWM_MASTER_TIM_NUMB == 9) void TIM1_BRK_TIM9_IRQHandler(void) #elif (PWM_MASTER_TIM_NUMB == 1) ¦¦ (PWM_MASTER_TIM_NUMB == 11) void TIM1_TRG_COM_TIM11_IRQHandler(void) #elif (PWM_MASTER_TIM_NUMB == 8) ¦¦ (PWM_MASTER_TIM_NUMB == 12) void TIM8_BRK_TIM12_IRQHandler(void) #elif (PWM_MASTER_TIM_NUMB == 8) ¦¦ (PWM_MASTER_TIM_NUMB == 14) void TIM8_TRG_COM_TIM14_IRQHandler(void) #endif { /* TIM_PWM_Handler */ Trace_PWM_TIM_Enter(); HAL_TIM_IRQHandler(&hpwm1.stim.htim); PWM_Handler(&hpwm1); Trace_PWM_TIM_Exit(); }

# ПРИЛОЖЕНИЕ Б

Листинг pwm.h

/** ************************************************************************** * @file pwm.h * @brief Заголовочный файл для модуля реализации ШИМ. * * @defgroup PWM * @brief PWM stuff * *************************************************************************/ #ifndef __PWM_H_ #define __PWM_H_ #include "control.h" extern uint32_t sin_table[SIN_TABLE_SIZE_MAX]; /** * @brief Transform int "XXxx" to float "XX.xx". * @param _int_ - число для преобразования. * @return _val_ - преобразованное число. */ #define int_to_percent(_int_) ((float)_int_/100) ///////////////////////////////////////////////////////////////////// ////////////////////////////---DEFINES---//////////////////////////// //----------------------------PWM HANDLE----------------------------// /** * @addtogroup PWM_HANDLE_DEFINES * @ingroup PWM * @brief Defines for access and processing elements of handle structure @{ */ /** * @brief Calc duration of maximum pulse in ticks. * @param _hpwm_ - указатель на хендл pwm. * @return _val_ - количество тиков кратчайшего импульса. */ #define PWM_Calc_Max_Duty(_hpwm_) (((_hpwm_)->PWM_MaxPulseDur)/(_hpwm_)->stim.sTickBaseUS) /** * @brief Calc duration of minimum pulse in ticks. * @param _hpwm_ - указатель на хендл pwm. * @return _val_ - количество тиков кратчайшего импульса. */ #define PWM_Calc_Min_Duty(_hpwm_) (((_hpwm_)->PWM_MinPulseDur)/(_hpwm_)->stim.sTickBaseUS) /** * @brief Calc Scale Koef for Table & AUTORELOAD REGISTER * @param _hpwm_ - указатель на хендл pwm. * @param _scale_ - верхняя граница диапазона значений. * @return _koef_ - коэффициент для масштабирования. * @details Данный макрос рассчитывает коэффициент для приведения значений с диапазоном [0,_scale_] к регистру автозагрузки с диапазоном [min, max], где min, max - минимальная и максимальная длительность импульса в тактах. */ #define PWM_Calc_Duty_Scale(_hpwm_, _scale_) ((float)(PWM_Calc_Max_Duty(_hpwm_)-PWM_Calc_Min_Duty(_hpwm_))/(_scale_)) /** * @brief Get Table Element Scaled corresponding to TIM ARR register * @param _hpwm_ - указатель на хендл pwm. * @param _ind_ - номер элемента из таблицы скважностей. * @return _val_ - масштабированный под регистры таймера значение. */ #define PWM_Get_Table_Element_Unsigned(_hpwm_,_ind_) (*((_hpwm_)->pDuty_Table_Origin+_ind_)) /** * @brief Get Table Element Shifted corresponding to TIM ARR register * @param _hpwm_ - указатель на хендл pwm. * @param _ind_ - номер элемента из таблицы скважностей. * @return _val_ - масштабированный под регистры таймера значение. * @details По сути такая же как PWM_Get_Table_Element_Unsigned но добавляется сдвиг вниз на одну амплитуду для учитывания знака. (если точнее, то сдвиг добавляется для компенсации сдвига вверх, для хранения в unsigned) * @note 0x8000*(_hpwm_)->Duty_Table_Scale - т.к. первая полуволна находится в диапазоне (0x8000-0xFFFF) вычитаем константу 0x8000 с масштабированием */ #define PWM_Get_Table_Element_Signed(_hpwm_,_ind_) ((int)PWM_Get_Table_Element_Unsigned(_hpwm_,_ind_)-0x8000) /** * @brief Scale Table Element corresponding to TIM ARR register * @param _hpwm_ - указатель на хендл pwm. * @param _element_ - число которое надо скейлнуть. * @details Если задана минимальная длительность импульса в тактах n, то регистр ARR заполняется так, что диапазон его значений будет [n, ARR-n] */ #define PWM_Scaled_Element(_hpwm_, _element_) ((_element_)*((_hpwm_)->Duty_Table_Scale)) /** * @brief Create pointer to slave PWM from pointer to void in PWM_HandleTypeDef. * @param _hpwm_ - указатель на хендл pwm. * @param _slavepwm_ - имя слейв pwm. * @return _pslavepwm_ - указатель на структуру PWM_SlaveHandleTypeDef. */ #define PWM_Set_pSlaveHandle(_hpwm_,_slavepwm_) ((PWM_SlaveHandleTypeDef *)_hpwm_->_slavepwm_) /** * @brief Copy setting from master TIM_SettingsTypeDef to slave TIM_SettingsTypeDef. * @param _hpwm_ - указатель на хендл pwm. * @param _set_ - имя настройки. */ #define PWM_Slave_CopyTimSetting(_hspwm_, _set_) ((_hspwm_)->stim._set_ = (_hspwm_)->hMasterPWM->stim._set_) /** PWM_HANDLE_DEFINES * @} */ //-----------------------------------------------------------------// //---------------------------TIMER REGS----------------------------// /** * @addtogroup PWM_TIMER_REGS_DEFINES * @ingroup PWM * @brief Defines for access to timers registers and processing handle structure @{ */ /** * @brief Get PWM autoreload value (max duty value). * @param _hpwm_ - указатель на хендл pwm. */ #define PWM_Get_Autoreload(_hpwm_) __HAL_TIM_GET_AUTORELOAD(&((_hpwm_)->stim.htim)) /** * @brief Get PWM Counter value. * @param _hpwm_ - указатель на хендл pwm. */ #define PWM_Get_Counter(_hpwm_) __HAL_TIM_GET_COUNTER(&((_hpwm_)->stim.htim)) /** * @brief Set PWM Counter value. * @param _hpwm_ - указатель на хендл pwm. * @param _val_ - значение, которое нужно записать в TIM->CNT. */ #define PWM_Set_Counter(_hpwm_, _val_) __HAL_TIM_SET_COUNTER(&((_hpwm_)->stim.htim), (_val_)) /** * @brief Get PWM Duty from channel 1. * @param _hpwm_ - указатель на хендл pwm. * @param _val_ - значение, которое нужно записать в Compare. */ #define PWM_Get_Compare1(_hpwm_) __HAL_TIM_GET_COMPARE(&((_hpwm_)->stim.htim), (_hpwm_)->sPWM_Channel1) /** * @brief Get PWM Duty from channel 2. * @param _hpwm_ - указатель на хендл pwm. * @param _val_ - значение, которое нужно записать в Compare. */ #define PWM_Get_Compare2(_hpwm_) __HAL_TIM_GET_COMPARE(&((_hpwm_)->stim.htim), (_hpwm_)->sPWM_Channel2) /** * @brief Set PWM Duty on Channel 1. * @param _hpwm_ - указатель на хендл pwm. * @param _val_ - значение, которое нужно записать в Compare. */ #define PWM_Set_Compare1(_hpwm_, _val_) __HAL_TIM_SET_COMPARE(&((_hpwm_)->stim.htim), (_hpwm_)->sPWM_Channel1, (_val_)) /** * @brief Set PWM Duty on Channel 2. * @param _hpwm_ - указатель на хендл pwm. * @param _val_ - значение, которое нужно записать в Compare. */ #define PWM_Set_Compare2(_hpwm_, _val_) __HAL_TIM_SET_COMPARE(&((_hpwm_)->stim.htim), (_hpwm_)->sPWM_Channel2, (_val_)) /** * @brief Set PWM Duty From PWM_Sine_Hz Percent * @param _hpwm_ - указатель на хендл pwm. * @param _channel_ - канал для выставления скважности. */ #define PWM_Set_Duty_From_Value(_hpwm_, _channel_) __HAL_TIM_SET_COMPARE(&((_hpwm_)->stim.htim), _channel_, ((_hpwm_)->PWM_Duty/100)*(PWM_Get_Autoreload(_hpwm_)+1)) /** * @brief Set PWM Duty From table * @param _hpwm_ - указатель на хендл pwm. * @param _channel_ - канал для выставления скважности. * @param _ind_ - номер элемента из таблицы скважностей. */ #define PWM_Set_Duty_From_Table(_hpwm_, _channel_, _ind_) __HAL_TIM_SET_COMPARE(&((_hpwm_)->stim.htim), _channel_, PWM_Scaled_Element( _hpwm_, PWM_Get_Table_Element_Unsigned((_hpwm_), (_ind_))+1) + PWM_Calc_Min_Duty(_hpwm_)) /** * @brief Set PWM Duty From PWM_Sine_Hz Percent * @param _hpwm_ - указатель на хендл pwm. * @param _channel_ - канал для выставления скважности. */ #define PWM_Set_SlaveDuty_From_Value(_hpwm_, _channel_) __HAL_TIM_SET_COMPARE(&((_hpwm_)->stim.htim), _channel_, ((_hpwm_)->hMasterPWM->PWM_Duty/100)*(PWM_Get_Autoreload(_hpwm_)+1)) /** * @brief Set PWM Duty From table * @param _hpwm_ - указатель на хендл pwm. * @param _channel_ - канал для выставления скважности. * @param _ind_ - номер элемента из таблицы скважностей. */ #define PWM_Set_SlaveDuty_From_Table(_hpwm_, _channel_, _ind_) __HAL_TIM_SET_COMPARE(&((_hpwm_)->stim.htim), _channel_, PWM_Scaled_Element((_hpwm_)->hMasterPWM, PWM_Get_Table_Element_Unsigned((_hpwm_)->hMasterPWM, (_ind_))+1) + PWM_Calc_Min_Duty((_hpwm_)->hMasterPWM)) /** TIMER REGS * @} */ //-----------------------------------------------------------------// //--------------------------MODE DEFINES---------------------------// /** * @addtogroup PWM_MODE_DEFINES * @ingroup PWM * @brief Defines for bits of @ref PWM_ModeTypeDef @{ */ #define PWM_DC_MODE_Pos (0) ///< Position of DC MODE bit in @ref PWM_ModeTypeDef #define PWM_BRIDGE_MODE_Pos (1) ///< Position of BRIDGE MODE bit in @ref PWM_ModeTypeDef #define PWM_PHASE_MODE_Pos (2) ///< Position of PHASE MODE bit in @ref PWM_ModeTypeDef #define PWM_ACTIVECHANNEL_MODE_Pos (3) ///< Position of ACTIVECHANNEL bits in @ref PWM_ModeTypeDef /** * @brief Bit for Sine/DC Mode * @par Modes: * @details - 0 - PWM has Sine form duty, * @details - 1 - PWM has DC duty */ #define PWM_DC_MODE (1<<(PWM_DC_MODE_Pos)) /** * @brief Bit for Bridge Mode * @par Modes: * @details - 0 - PWM generated only on one channel ( @ref PWM_ACTIVECHANNEL_MODE) * @details - 1 - PWM generated at two channels of each phase ( @ref PWM_PHASE_MODE) */ #define PWM_BRIDGE_MODE (1<<(PWM_BRIDGE_MODE_Pos)) /** * @brief Bit for 3-Phase Mode * @par Modes: * @details - 0 - PWM generated only at phase "A" * @details - 1 - PWM generated at three phase "A", "B", "C" */ #define PWM_PHASE_MODE (1<<(PWM_PHASE_MODE_Pos)) /** * @brief Bit for 3-Phase Mode * @par Modes: * @details - 0 - PWM doesnt generated * @details - 1-6 - PWM generated on corresponting channel № "PWM_ACTIVECHANNEL_MODE" */ #define PWM_ACTIVECHANNEL_MODE (0x7<<(PWM_ACTIVECHANNEL_MODE_Pos)) // 0 - One channel PWM doesnt generated, 1 - PWM generated on corresponting channel № "PWM_ACTIVECHANNEL_MODE" /** * @brief Mode: PWM generation disabled */ #define PWM_ACTIVECHANNEL_DISABLE (0x0<<(PWM_ACTIVECHANNEL_MODE_Pos)) /** * @brief Mode: PWM generated on channel 1 (Phase A channel 0) */ #define PWM_ACTIVECHANNEL_A0 (0x1<<(PWM_ACTIVECHANNEL_MODE_Pos)) /** * @brief Mode: PWM generated on channel 2 (Phase A channel 1) */ #define PWM_ACTIVECHANNEL_A1 (0x2<<(PWM_ACTIVECHANNEL_MODE_Pos)) /** * @brief Mode: PWM generated on channel 3 (Phase B channel 0) */ #define PWM_ACTIVECHANNEL_B0 (0x3<<(PWM_ACTIVECHANNEL_MODE_Pos)) /** * @brief Mode: PWM generated on channel 4 (Phase B channel 1) */ #define PWM_ACTIVECHANNEL_B1 (0x4<<(PWM_ACTIVECHANNEL_MODE_Pos)) /** * @brief Mode: PWM generated on channel 5 (Phase C channel 0) */ #define PWM_ACTIVECHANNEL_C0 (0x5<<(PWM_ACTIVECHANNEL_MODE_Pos)) /** * @brief Mode: PWM generated on channel 6 (Phase C channel 1) */ #define PWM_ACTIVECHANNEL_C1 (0x6<<(PWM_ACTIVECHANNEL_MODE_Pos)) /** * @brief Read current PWM mode from handle */ #define PWM_Get_Mode(_hpwm_, _mode_) ((_hpwm_)->sPWM_Mode&(_mode_)) /** * @brief Structure for PWM modes. */ typedef enum { // SINE MODES PWM_SINE_SINGLE = 0, /*!< set pwm duty from table with PWM_Sine_Hz Hz */ PWM_SINE_BRIDGE = PWM_BRIDGE_MODE, /*!< set pwm duty from table with PWM_Sine_Hz Hz on two channels (positive and negative halfes) */ PWM_SINE_3PHASE = PWM_BRIDGE_MODE¦PWM_PHASE_MODE, /*!< set pwm table duty on three phase with PWM_Sine_Hz Hz */ // DC MODES PWM_DC_SINGLE = PWM_DC_MODE, /*!< set pwm duty PWM_Sine_Hz (in percent) on one channel ( @ref PWM_ACTIVECHANNEL_MODE) */ PWM_DC_BRIDGE = PWM_DC_MODE¦PWM_BRIDGE_MODE, /*!< set pwm duty PWM_Duty (in percent) on two channel with PWM_Sine_Hz Hz */ PWM_DC_3PHASE = PWM_DC_MODE¦PWM_BRIDGE_MODE¦PWM_PHASE_MODE, /*!< set pwm PWM_Duty duty on three phase with PWM_Sine_Hz Hz, with requested shift */ // Variables of signel channel modes PWM_SINE_SINGLE_A0 = PWM_ACTIVECHANNEL_A0, /*!< set sine pwm on Phase A channel 0 */ PWM_SINE_SINGLE_A1 = PWM_ACTIVECHANNEL_A1, /*!< set sine pwm on Phase A channel 1 */ PWM_SINE_SINGLE_B0 = PWM_ACTIVECHANNEL_B0, /*!< set sine pwm on Phase B channel 0 */ PWM_SINE_SINGLE_B1 = PWM_ACTIVECHANNEL_B1, /*!< set sine pwm on Phase B channel 1 */ PWM_SINE_SINGLE_C0 = PWM_ACTIVECHANNEL_C0, /*!< set sine pwm on Phase C channel 0 */ PWM_SINE_SINGLE_C1 = PWM_ACTIVECHANNEL_C1, /*!< set sine pwm on Phase C channel 1 */ PWM_SINE_BRIDGE_A0 = PWM_DC_MODE¦PWM_ACTIVECHANNEL_A0, /*!< set DC pwm on Phase A channel 0) */ PWM_SINE_BRIDGE_A1 = PWM_DC_MODE¦PWM_ACTIVECHANNEL_A1, /*!< set DC pwm on Phase A channel 1) */ PWM_SINE_BRIDGE_B0 = PWM_DC_MODE¦PWM_ACTIVECHANNEL_B0, /*!< set DC pwm on Phase B channel 0) */ PWM_SINE_BRIDGE_B1 = PWM_DC_MODE¦PWM_ACTIVECHANNEL_B1, /*!< set DC pwm on Phase B channel 1) */ PWM_SINE_BRIDGE_C0 = PWM_DC_MODE¦PWM_ACTIVECHANNEL_C0, /*!< set DC pwm on Phase C channel 0) */ PWM_SINE_BRIDGE_C1 = PWM_DC_MODE¦PWM_ACTIVECHANNEL_C1, /*!< set DC pwm on Phase C channel 1) */ }PWM_ModeTypeDef; /** PWM_MODE_DEFINES * @} */ //-----------------------------------------------------------------// //-----------------------HANDLE STRUCTURES-------------------------// /** * @addtogroup PWM_HANDLE_STRUCTURES * @ingroup PWM * @brief Structures for handling PWM @{ */ /** * @brief Structure for PWM modbus coils. */ typedef struct // PWM_ModeCoilsTypeDef { unsigned reserved:1; /*!< reserved bits */ unsigned DC:1; /*!< Sine/DC mode*/ unsigned BRIDGE:1; /*!< Bridge mode */ unsigned PHASE:1; /*!< minimum pulse duration for PWM in us*/ unsigned POLARITY:1; /*!< minimum pulse duration for PWM in us*/ unsigned ACTIVECHANNEL:3; /*!< minimum pulse duration for PWM in us*/ }PWM_ModeCoilsTypeDef; /** * @brief Structure for PWM modbus registers. */ typedef struct // PWM_ModeRegsTypeDef { uint16_t Sine_Hz; /*!< frequency of sine */ uint16_t Duty; /*!< duty of dc pwm */ uint16_t TIMFreqHz; /*!< frequency of pwm timer */ uint16_t MaxPulseDur; /*!< maximum pulse duration for PWM in us*/ uint16_t MinPulseDur; /*!< minimum pulse duration for PWM in us*/ uint16_t DeadTime; /*!< dead-Time between switches half waves (channels) in us */ uint16_t TickBasePrescaler; /*!< tickbase prescaler for pwm timer */ uint16_t TableSize; /*!< size of sine table */ }PWM_ModeRegsTypeDef; /** * @brief Structure for PWM config. */ typedef struct // PWM_ConfigTypeDef { PWM_ModeCoilsTypeDef *PWM_Mode; /*!< Mode of PWM: modbus coils */ PWM_ModeRegsTypeDef *PWM_Settings; /*!< Parameters of PWM: modbus registers */ }PWM_ConfigTypeDef; /** * @brief Handle for ramp. */ typedef struct { /* Controller gains */ arm_pid_instance_f32 pid; /*!< Parameters of PID-regulator */ /* Output limits */ float limMin; /*!< Limit of PID-regulator: max rate of increase */ float limMax; /*!< Limit of PID-regulator: max rate of decrease */ /* Integrator limits */ float limMinInt; /*!< Limit of integrator: max rate of increase */ float limMaxInt; /*!< Limit of integrator: max rate of decrease */ /* Sample time (in seconds) */ float SampleT; /*!< Sample time of regulator (usualy 1/TIM_Hz that calls function of update PID-regulator) */ } PWM_RampHandleTypeDef; #define PWM_PID_state_prevOut 1 ///< index of handle of ramp for PWM Sine Hz #define PWM_PID_state_Input 0 ///< index of handle of ramp for PWM Sine Hz #define PWM_PID_state_Output 2 ///< index of handle of ramp for PWM Sine Hz #define PWM_RampSineHz_Ind 0 ///< index of handle of ramp for PWM Sine Hz #define PWM_RampPWMDuty_Ind 1 ///< index of handle of ramp for PWM Duty /** * @brief Handle for PWM. * @note Prefixes: h - handle, s - settings, f - flag */ typedef struct // PWM_HandleTypeDef { /* PWM VARIABLES */ PWM_ConfigTypeDef sPWM_Config; /*!< PWM modbus config */ PWM_ModeTypeDef sPWM_Mode; /*!< PWM current mode */ float PWM_Sine_Hz; /*!< Current frequency */ float PWM_Duty; /*!< Current duty for DC bridge mode */ uint16_t PWM_MaxPulseDur; /*!< Current maximum pulse duration for PWM in tickbase*/ uint16_t PWM_MinPulseDur; /*!< Current minimum pulse duration for PWM in tickbase*/ uint16_t PWM_DeadTime; /*!< Current dead-Time between switches half waves (channels) in us */ PWM_RampHandleTypeDef hramp[2]; /*!< Handle for ramp */ uint8_t PWM_enHardware; /*!< Enable PWM output on transistor */ /* SETTINGS FOR TIMER */ TIM_SettingsTypeDef stim; /*!< Settings for TIM */ TIM_OC_InitTypeDef sConfigPWM; /*!< Settings for PWM oc channel */ TIM_OC_InitTypeDef sConfigTrigger; /*!< Settings for Trigger oc channel */ unsigned fActiveChannel; /*!< Flag for active oc channel: 0 - first channel, 1 - second channel */ uint16_t sPWM_Channel1; /*!< Instance of first channel */ uint16_t sPWM_Channel2; /*!< Instance of second channel */ /* VARIABLES FOR TABLE DUTY PARAMETERS */ uint32_t *pDuty_Table_Origin; /*!< Pointer to table of pwm duties */ uint32_t Duty_Table_Size; /*!< Size of duty table */ float Duty_Table_Ind; /*!< Current ind of duty table */ float Duty_Table_Scale; /*!< Scale for TIM ARR register */ /* SETTIGNS FOR PWM OUTPUT */ GPIO_TypeDef *GPIOx; /*!< GPIO port for PWM output */ uint32_t GPIO_PIN_X1; /*!< GPIO pin for PWM output */ uint32_t GPIO_PIN_X2; /*!< GPIO pin for PWM output (in bridge mode) */ /* SLAVES PWM */ void *hpwm2; /*!< Pointer to handle of slave PWM 2 */ void *hpwm3; /*!< Pointer to handle of slave PWM 3 */ }PWM_HandleTypeDef; extern PWM_HandleTypeDef hpwm1; ///< Master PWM handle /** * @brief Handle for Slave PWM. * @note Prefixes: h - handle, s - settings, f - flag */ typedef struct // PWM_SlaveHandleTypeDef { /* MASTER PWM*/ PWM_HandleTypeDef *hMasterPWM; /*!< master pwm handle */ /* SETTINGS FOR TIMER */ TIM_SettingsTypeDef stim; /*!< slave tim handle */ unsigned fActiveChannel; /*!< flag for active oc channel: 0 - first channel, 1 - second channel */ uint16_t sPWM_Channel1; /*!< instance of first channel */ uint16_t sPWM_Channel2; /*!< instance of second channel */ /* VARIABLES FOR TABLE DUTY PARAMETERS */ float Duty_Shift_Ratio; /*!< Ratio of table shift: 0.5 shift - shift = Table_Size/2 */ /* SETTIGNS FOR PWM OUTPUT */ GPIO_TypeDef *GPIOx; /*!< GPIO port for PWM output */ uint32_t GPIO_PIN_X1; /*!< GPIO pin for PWM output */ uint32_t GPIO_PIN_X2; /*!< GPIO pin for PWM output (second half wave) */ }PWM_SlaveHandleTypeDef; extern PWM_SlaveHandleTypeDef hpwm2; ///< Slave PWM handle extern PWM_SlaveHandleTypeDef hpwm3; ///< Slave PWM handle /** PWM_HANDLE_STRUCTURES * @} */ //-----------------------------------------------------------------// //--------------------------------PWM FUNCTIONS---------------------------------- /** * @addtogroup PWM_FUNCTIONS * @ingroup PWM * @brief Function for control and configuring PWM */ // MASTER PWM FUNCTIONS /** * @addtogroup PWM_MAIN_FUNCTIONS * @ingroup PWM_FUNCTIONS * @brief Function for controling PWM * @{ */ /* PWM Handler */ void PWM_Handler(PWM_HandleTypeDef *hpwm); /* Form PWM on Single Channel (DC or Sine PWM). Slave PWM disabled */ void PWM_SingleChannel_Mode(PWM_HandleTypeDef *hpwm, uint16_t rotate_ind_A, uint8_t SineOrDC); /* Form Bridge Sine PWM. Forming Slave PWM included */ void PWM_Sine_Bridge_Mode(PWM_HandleTypeDef *hpwm, uint16_t rotate_ind_A, uint16_t rotate_ind_B, uint16_t rotate_ind_C); /* Form Bridge DC PWM. Forming Slave PWM included */ void PWM_DC_Bridge_Mode(PWM_HandleTypeDef *hpwm, uint16_t rotate_ind_A, uint16_t rotate_ind_B, uint16_t rotate_ind_C); /* Getting ind for Duty Table */ uint32_t PWM_Get_Duty_Table_Ind(PWM_HandleTypeDef *hpwm, float FreqTIM); /* Create Dead Time when switches channels */ void PWM_CreateDeadTime(PWM_HandleTypeDef *hpwm, float *LocalDeadTimeCnt, unsigned *LocalActiveChannel); /** PWM_MAIN_FUNCTIONS * @} */ // SLAVE PWM FUNCTIONS /** * @addtogroup PWM_SLAVE_FUNCTIONS * @ingroup PWM_FUNCTIONS * @brief Function for controling slaves PWM * @{ */ /* Set Duty from table on Slave PWM at one channel by sin_ind of the Master PWM */ uint16_t PWM_SlavePhase_Calc_TableInd(PWM_SlaveHandleTypeDef *hspwm, uint16_t sin_ind); /* Check is all Slave channels works properly */ void PWM_SlavePhase_Check_Channels(PWM_SlaveHandleTypeDef *hspwm); /* Create Dead Time for Slave PWM when switches channels */ void PWM_SlavePhase_CreateDeadTime(PWM_SlaveHandleTypeDef *hspwm, float *LocalDeadTimeCnt, unsigned *LocalActiveChannel); /** PWM_SLAVE_FUNCTIONS * @} */ /** * @addtogroup PWM_CONFIG_FUNCTIONS * @ingroup PWM_FUNCTIONS * @brief Function for initializing and re-configuring PWM * @{ */ /* Update PWM parameters */ void PWM_Update_Params(PWM_HandleTypeDef *hpwm); /* PID for ramp. */ void PWM_Ramp_ControlValue(PWM_RampHandleTypeDef *hramp, float *PID_Output, float PID_Input); /* Update PID parameters */ void PWM_Ramp_UpdateParams(PWM_RampHandleTypeDef *hramp, int32_t flagReset); /* Reset PID for ramp */ void PWM_Ramp_Reset(PWM_RampHandleTypeDef *hramp); /* Filling table with one period of sinus values */ uint32_t PWM_Fill_Sine_Table(PWM_HandleTypeDef *hpwm, uint32_t table_size); //----------------------------------init func------------------------------------ /* First set up of PWM Two Channel */ void PWM_Sine_FirstInit(void); void PWM_SynchInit(void); /* reInitialization of PWM TIM */ void PWM_Sine_ReInit(PWM_HandleTypeDef *hpwm); /* Initialization of Slave PWM TIM */ void PWM_SlavePhase_Init(PWM_SlaveHandleTypeDef *hspwm); /* reInitialization of Slave PWM TIM */ void PWM_SlavePhase_reInit(PWM_SlaveHandleTypeDef *hspwm); /* Calc and update new Duty Table Scale */ void PWM_Update_DutyTableScale(PWM_HandleTypeDef *hpwm); /** PWM_CONFIG_FUNCTIONS * @} */ //-----------------------------------------------------------------// #endif // __PWM_H_

# ПРИЛОЖЕНИЕ В

Листинг interface.c

/** ************************************************************************** * @file interface.c * @brief Модуль для реализации интерфейса (енкодер и LCD дисплей) *************************************************************************/ #include "interface.h" #include "pwm.h" TIM_SettingsTypeDef TIM_ENCODER = {0}; Menu_TypeDef MENU; extern I2C_HandleTypeDef hi2c1; SPI_SettingsTypeDef sspi_disp; extern PWM_HandleTypeDef hpwm1; Interface_HandleTypeDef hinterface; //-----------------ENCODER FUNCTIONS----------------// /** * @brief Update Encoder. * @param hinterface - handle for interface. * @note Encoder control menu of parameters and their values. * @note This function call all functions below in section @ref INTERFACE_ENCODER_FUNCTIONS. */ void Update_Encoder(Interface_HandleTypeDef *hinterface) { // если энкодера не в режиме точной настройки) if(hinterface->FineChanging == 0) { // Нажатие кнопки if((~hinterface->henc.GPIOx->IDR)&hinterface->henc.GPIO_PIN_SW) { msDelay(50); // задержка для дребезга if((~hinterface->henc.GPIOx->IDR)&hinterface->henc.GPIO_PIN_SW) // если кнопка действительно нажата { // Первый клик if(HAL_GetTick() - hinterface->Switch_prevTick > hinterface->DoubleClick_Timeout) // слишком большая задержка после предыдущего нажатия { Encoder_SingleClick(hinterface); } // Дабл клик else if(HAL_GetTick() - hinterface->Switch_prevTick < hinterface->DoubleClick_Timeout) // слишком маленькая задержка после предыдущего нажатия { Encoder_DoubleClick(hinterface); } } } } // если екодер в режиме точной настройки) if(hinterface->FineChanging == 1) { if(((~hinterface->henc.GPIOx->IDR)&hinterface->henc.GPIO_PIN_SW) == 0) // ожидание отпускания кнопки { msDelay(50); if(((~hinterface->henc.GPIOx->IDR)&hinterface->henc.GPIO_PIN_SW) == 0) // если кнопка действительно отпущена { hinterface->FineChanging = 0; // отключение режима точной настройки } } } // запись считанных данных с енкодера Encoder_UpdateVarsFromEncoder(hinterface); } void Encoder_SingleClick(Interface_HandleTypeDef *hinterface) { hinterface->Switch_prevTick = HAL_GetTick(); // сохраняем время нажатия для регистрации долгого удержания кнопки если оно будет hinterface->Encoder_Shdw = hinterface->henc.htim->Instance->CNT; while((~hinterface->henc.GPIOx->IDR)&hinterface->henc.GPIO_PIN_SW) // ожидаем её отпускания { // если долгое удержание if(Encoder_HoldingSwitch(hinterface)) { return; // выход из функции } // если кнопка не отпускается, и енкодер начинает регулироваться if(Encoder_HoldingAndRotateEncoder(hinterface)) { return; // выход из функции } msDelay(1); } // когда кнопка отпущена Encoder_SwitchRelease(hinterface); } void Encoder_DoubleClick(Interface_HandleTypeDef *hinterface) { // переключение на другое меню hinterface->MenuNumber++; if(hinterface->MenuNumber > COILS_MENU) { hinterface->MenuNumber = 0; } hinterface->StartDispItem = 0; hinterface->CurrentSelection = NOTHING_SELECTED; // if (hinterface->MenuNumber == 0) // hinterface->CurrentSelection = NOTHING_SELECTED; // else if (hinterface->MenuNumber == 1) // hinterface->CurrentSelection = NOTHING_COIL_SELECTED; } /** * @brief Holding Switch action. * @param hinterface - handle for interface. * @note This called when switch is holding for 5 second. */ uint8_t Encoder_HoldingSwitch(Interface_HandleTypeDef *hinterface) { if(HAL_GetTick() - hinterface->Switch_prevTick > 5000) // если долго удержание - 5 секунд { #ifdef LCD_I2C // запрос на реинициализацю дисплея hinterface->hlcd.LCD_REINIT = 1; #endif return 1; // 1 для выхода из цикла } return 0; // 0 чтобы остаться в цикле } /** * @brief Holding Switch and Rotate encoder action. * @param hinterface - handle for interface. * @note This called when switch is holding and encoder starts rotating. */ uint8_t Encoder_HoldingAndRotateEncoder(Interface_HandleTypeDef *hinterface) { if (hinterface->Encoder_Shdw != hinterface->henc.htim->Instance->CNT) // если кнопка не отпускается, и енкодер начинает регулироваться { hinterface->FineChanging = 1; // переходим в режим точной настройки return 1; // 1 для выхода из цикла } return 0; // 0 чтобы остаться в цикле } /** * @brief Release Switch action. * @param hinterface - handle for interface. * @note This called when switch is clicked . */ void Encoder_SwitchRelease(Interface_HandleTypeDef *hinterface) { uint32_t *displayingitems; if(hinterface->MenuNumber == REGISTERS_MENU) displayingitems = &hinterface->Displaying.Regs.all; else if(hinterface->MenuNumber == COILS_MENU) displayingitems = &hinterface->Displaying.Coils.all; if(hinterface->FineChanging == 0) // если энкодер не перешел в режим точной настройки) { hinterface->Switch_prevTick = HAL_GetTick(); // сохраняем время отжатия для регистрации даблклика если он будет // hinterface->CurrentSelection++; // переключаем параметр для настройки do{ hinterface->CurrentSelection++; // проверка на конец списка меню if(hinterface->MenuNumber == REGISTERS_MENU) { // в меню регистров переключаем до размера меню регистров if(hinterface->CurrentSelection > Menu_Regs_Size) { // сброс обратно до меню hinterface->CurrentSelection = NOTHING_SELECTED; break; } } else if(hinterface->MenuNumber == COILS_MENU) // COILS_MENU { // в меню коилов переключаем до размера меню коилов if(hinterface->CurrentSelection - NOTHING_COIL_SELECTED > Menu_Coil_Size) { // сброс обратно до меню hinterface->CurrentSelection = NOTHING_COIL_SELECTED; break; } } } // пропускаем все настройки меню, которые не отображаются н дисплее while((*displayingitems&(1<<(hinterface->CurrentSelection-1))) == 0); } } /** * @brief Update params variables from encoder rotate. * @param hinterface - handle for interface. * @note This function call @ref Encoder_SetUint16Value. */ void Encoder_UpdateVarsFromEncoder(Interface_HandleTypeDef *hinterface) { static uint16_t coef_value; if(hinterface->FineChanging == 0) // если енкодер не регулируется coef_value = 100; else coef_value = 2; hinterface->henc.Encoder_Diff = (int32_t)hinterface->henc.htim->Instance->CNT - hinterface->Encoder_Shdw; if(hinterface->FineChanging == 0) { } int32_t tmpreg; uint8_t scrolmenu_flag = 0; if(hinterface->henc.Encoder_Diff != 0) { if(hinterface->MenuNumber == REGISTERS_MENU) { switch(hinterface->CurrentSelection) { case PWM_SINE_HZ_SELECTED: Encoder_SetUint16Value(&hinterface->henc, &hpwm1.sPWM_Config.PWM_Settings->Sine_Hz, coef_value, 0, 0xFFFF); break; case PWM_DUTY_SELECTED: Encoder_SetUint16Value(&hinterface->henc, &hpwm1.sPWM_Config.PWM_Settings->Duty, coef_value, 0, 10000); break; case PWM_HZ_SELECTED: Encoder_SetUint16Value(&hinterface->henc, &hpwm1.sPWM_Config.PWM_Settings->TIMFreqHz, coef_value, 0, 0xFFFF); break; case PWM_MAXPULSE_SELECTED: // if max duration in equal min duration bot not zero: set minimum as 0. for "if" statement later if((hpwm1.sPWM_Config.PWM_Settings->MaxPulseDur <= hpwm1.PWM_MinPulseDur) && (hpwm1.sPWM_Config.PWM_Settings->MaxPulseDur != 0) ) Encoder_SetUint16Value(&hinterface->henc, &hpwm1.sPWM_Config.PWM_Settings->MaxPulseDur, ((coef_value>50)? coef_value/10 : coef_value), 0, 0xFFFF); // if max duration in some other value: set minimum as min duration else Encoder_SetUint16Value(&hinterface->henc, &hpwm1.sPWM_Config.PWM_Settings->MaxPulseDur, ((coef_value>50)? coef_value/10 : coef_value), hpwm1.PWM_MinPulseDur, 0xFFFF); // if max duration are less than min duration: set max duration to defalut (zero) if(hpwm1.sPWM_Config.PWM_Settings->MaxPulseDur < hpwm1.PWM_MinPulseDur) hpwm1.sPWM_Config.PWM_Settings->MaxPulseDur = 0; // automcaticaly calc max duration corresponding to ARR register break; case PWM_MINPULSE_SELECTED: Encoder_SetUint16Value(&hinterface->henc, &hpwm1.sPWM_Config.PWM_Settings->MinPulseDur, ((coef_value>50)? coef_value/10 : coef_value), 0, hpwm1.PWM_MaxPulseDur); break; case PWM_DEADTIME_SELECTED: Encoder_SetUint16Value(&hinterface->henc, &hpwm1.sPWM_Config.PWM_Settings->DeadTime, coef_value, 0, 0xFFFF); break; case PWM_TICKBASEPRESC_SELECTED: Encoder_SetUint16Value(&hinterface->henc, &hpwm1.sPWM_Config.PWM_Settings->TickBasePrescaler, 1, 0, 999); break; case NOTHING_SELECTED: if(hinterface->henc.Encoder_Diff > 0) hinterface->StartDispItem += 1; else if(hinterface->henc.Encoder_Diff < 0) hinterface->StartDispItem -= 1; if(hinterface->StartDispItem < 0) hinterface->StartDispItem = 0; break; default: scrolmenu_flag = 1; break; } } else if (hinterface->MenuNumber == COILS_MENU) { switch(hinterface->CurrentSelection) { case PWM_COIL_DC_SELECTED: if(hinterface->henc.Encoder_Diff) hpwm1.sPWM_Config.PWM_Mode->DC = ~hpwm1.sPWM_Config.PWM_Mode->DC; break; case PWM_COIL_BRIDGE_SELECTED: if(hinterface->henc.Encoder_Diff) hpwm1.sPWM_Config.PWM_Mode->BRIDGE = ~hpwm1.sPWM_Config.PWM_Mode->BRIDGE; break; case PWM_COIL_PHASE_SELECTED: if(hinterface->henc.Encoder_Diff) hpwm1.sPWM_Config.PWM_Mode->PHASE = ~hpwm1.sPWM_Config.PWM_Mode->PHASE; break; case PWM_COIL_POLARITY_SELECTED: if(hinterface->henc.Encoder_Diff) hpwm1.sPWM_Config.PWM_Mode->POLARITY = ~hpwm1.sPWM_Config.PWM_Mode->POLARITY; break; case PWM_COIL_ACTIVECHANNEL_SELECTED: tmpreg = hpwm1.sPWM_Config.PWM_Mode->ACTIVECHANNEL; tmpreg += (hinterface->henc.Encoder_Diff); if (tmpreg < 0) hpwm1.sPWM_Config.PWM_Mode->ACTIVECHANNEL = 0; else if (tmpreg > 6) hpwm1.sPWM_Config.PWM_Mode->ACTIVECHANNEL = 6; else if (hinterface->henc.Encoder_Diff > 0) hpwm1.sPWM_Config.PWM_Mode->ACTIVECHANNEL++; else if (hinterface->henc.Encoder_Diff < 0) hpwm1.sPWM_Config.PWM_Mode->ACTIVECHANNEL--; break; default: scrolmenu_flag = 1; break; } } if(scrolmenu_flag) { if(hinterface->henc.Encoder_Diff > 0) hinterface->StartDispItem += 1; else if(hinterface->henc.Encoder_Diff < 0) hinterface->StartDispItem -= 1; if(hinterface->StartDispItem < 0) hinterface->StartDispItem = 0; } } hinterface->Encoder_Shdw = hinterface->henc.htim->Instance->CNT; } /** * @brief Set uint16_t value from from encoder rotate. * @param hinterface - handle for interface. * @note Target value is ranged between min_value and max_value. */ void Encoder_SetUint16Value(TIM_EncoderTypeDef *henc, uint16_t *TargetValue, uint16_t coef, int32_t min_value, int32_t max_value) { int32_t tmpreg; tmpreg = *TargetValue; tmpreg += ((henc->Encoder_Diff*coef)/2); if (tmpreg < min_value) *TargetValue = min_value; else if (tmpreg > max_value) *TargetValue = max_value; else *TargetValue = tmpreg; } /** * @brief Set uint8_t value from from encoder rotate. * @param hinterface - handle for interface. * @note @ref Encoder_SetUint16Value. */ void Encoder_SetUint8Value(TIM_EncoderTypeDef *henc, uint8_t *TargetValue, uint16_t coef, int32_t min_value, int32_t max_value) { int32_t tmpreg; tmpreg = *TargetValue; tmpreg += ((henc->Encoder_Diff*coef)/2); if (tmpreg < min_value) *TargetValue = min_value; else if (tmpreg > max_value) *TargetValue = max_value; else *TargetValue = tmpreg; } //---------------LCD DISPLAY FUNCTIONS--------------// /** * @brief Update Active Items in Menu Structure for displaying. * @param hinterface - handle for interface. * @note Update corresponding to current mode and stop displaying unnecessary items. * @note This called from StartMainTask */ void Update_ActiveMenuItems(Interface_HandleTypeDef *hinterface) { if(hinterface->MenuNumber == REGISTERS_MENU) { // RegsMenu_HandleTypeDef *RegsMenu = (RegsMenu_HandleTypeDef *)&hinterface->Menu.Regs; if(hpwm1.sPWM_Config.PWM_Mode->BRIDGE ¦¦ (hpwm1.sPWM_Config.PWM_Mode->DC == 0)) hinterface->Menu.Regs.Items.PWM_Sine_Hz = 1; else hinterface->Menu.Regs.Items.PWM_Sine_Hz = 0; if(hpwm1.sPWM_Config.PWM_Mode->DC) hinterface->Menu.Regs.Items.PWM_Duty = 1; else hinterface->Menu.Regs.Items.PWM_Duty = 0; hinterface->Menu.Regs.Items.PWM_Hz = 1; hinterface->Menu.Regs.Items.PWM_MaxPulseDur = 1; hinterface->Menu.Regs.Items.PWM_MinPulseDur = 1; if(hpwm1.sPWM_Config.PWM_Mode->BRIDGE) hinterface->Menu.Regs.Items.PWM_DeadTime = 1; else hinterface->Menu.Regs.Items.PWM_DeadTime = 0; hinterface->Menu.Regs.Items.PWM_TickBasePresc = 1; } else if(hinterface->MenuNumber == COILS_MENU) { // CoilsMenu_HandleTypeDef *CoilsMenu = (CoilsMenu_HandleTypeDef *)&hinterface->Menu[COILS_MENU]; hinterface->Menu.Coils.Items.PWM_DC = 1; hinterface->Menu.Coils.Items.PWM_BRIDGE = 1; if(hpwm1.sPWM_Config.PWM_Mode->BRIDGE) { hinterface->Menu.Coils.Items.PWM_PHASE = 1; hinterface->Menu.Coils.Items.PWM_ACTIVECHANNEL = 0; } else { hinterface->Menu.Coils.Items.PWM_PHASE = 0; hinterface->Menu.Coils.Items.PWM_ACTIVECHANNEL = 1; } hinterface->Menu.Coils.Items.PWM_POLARITY = 1; } } /** * @brief Update LCD display. * @param hinterface - handle for interface. * @note LCD displaying parameters of PWM. * @note This call all functions below in section @ref LCD DISPLAY FUNCTIONS. */ void Update_LCDDisplay(Interface_HandleTypeDef *hinterface) { // clear display #ifdef LCD_I2C LCD_Check(&hinterface->hlcd); LCD_Send_CMD(&hinterface->hlcd, 0x01); msDelay(hinterface->LCD_ClearDelay); #endif #ifdef LCD_SPI SPIDisp_Clean_Buffer_Frame(); #endif hinterface->disp_cnt = 0 - hinterface->StartDispItem; if(hinterface->MenuNumber == 0) hinterface->disp_cnt = LCD_DisplayMenuRegisters(hinterface); else hinterface->disp_cnt = LCD_DisplayMenuCoils(hinterface); vTaskDelayUntil(&hinterface->LCD_prevUpdateTick, hinterface->LCD_UpdateDelay); #ifdef LCD_SPI // update frame SPIDisp_Update(); #endif // if more items can be showed item displayed - decrease start menu item if((hinterface->StartDispItem > 0) && (hinterface->disp_cnt < hinterface->hlcd.LCD_Rows)) { hinterface->StartDispItem -= 1; } } /** * @brief Display all coils menu at LCD Display. * @param hinterface - handle for interface. * @return disp_cnt - how many string is displayed. * @note This call LCD_PrintString and LCD_UpdateString. */ uint8_t LCD_DisplayMenuCoils(Interface_HandleTypeDef *hinterface) { uint8_t disp_flag = 0; uint8_t line = 0; for(int i = 0; i < Menu_Coil_Size; i++) { switch(hinterface->Menu.Coils.all&(1<<i)) { // delete item from displaying if it isnt displaying case 0: disp_flag = 0; break; case Menu_PWM_DC: disp_flag = LCD_PrintString(hinterface, PWM_COIL_DC_SELECTED, Print_PWM_DC); break; case Menu_PWM_BRIDGE: disp_flag = LCD_PrintString(hinterface, PWM_COIL_BRIDGE_SELECTED, Print_PWM_BRIDGE); break; case Menu_PWM_PHASE: disp_flag = LCD_PrintString(hinterface, PWM_COIL_PHASE_SELECTED, Print_PWM_PHASE); break; case Menu_PWM_POLARITY: disp_flag = LCD_PrintString(hinterface, PWM_COIL_POLARITY_SELECTED, Print_PWM_POLARITY); break; case Menu_PWM_ACTIVECHANNEL: disp_flag = LCD_PrintString(hinterface, PWM_COIL_ACTIVECHANNEL_SELECTED, Print_PWM_ACTIVECHANNEL); break; } disp_flag = LCD_UpdateString(hinterface, disp_flag); // update string // update displaying items srtuct if(disp_flag) hinterface->Displaying.Coils.all ¦= (1<<i); else hinterface->Displaying.Coils.all &= ~(1<<i); } return hinterface->disp_cnt; } /** * @brief Display all registers menu at LCD Display. * @param hinterface - handle for interface. * @return disp_cnt - how many string is displayed. * @note This call LCD_PrintString and LCD_UpdateString. */ uint8_t LCD_DisplayMenuRegisters(Interface_HandleTypeDef *hinterface) { uint8_t disp_flag = 0; uint8_t line = 0; for(int i = 0; i < Menu_Regs_Size; i++) { switch(hinterface->Menu.Regs.all&(1<<i)) { // delete item from displaying if it isnt displaying case 0: disp_flag = 0; break; case Menu_PWM_Sine_Hz: disp_flag = LCD_PrintString(hinterface, PWM_SINE_HZ_SELECTED, Print_PWM_Sine_Hz); break; case Menu_PWM_Duty: disp_flag = LCD_PrintString(hinterface, PWM_DUTY_SELECTED, Print_PWM_Duty); break; case Menu_PWM_Hz: disp_flag = LCD_PrintString(hinterface, PWM_HZ_SELECTED, Print_PWM_Hz); break; case Menu_PWM_MaxPulseDur: disp_flag = LCD_PrintString(hinterface, PWM_MAXPULSE_SELECTED, Print_PWM_MaxPulseDur); break; case Menu_PWM_MinPulseDur: disp_flag = LCD_PrintString(hinterface, PWM_MINPULSE_SELECTED, Print_PWM_MinPulseDur); break; case Menu_PWM_DeadTime: disp_flag = LCD_PrintString(hinterface, PWM_DEADTIME_SELECTED, Print_PWM_DeadTime); break; case Menu_PWM_TickBasePresc: disp_flag = LCD_PrintString(hinterface, PWM_TICKBASEPRESC_SELECTED, Print_PWM_TickBasePresc); break; } disp_flag = LCD_UpdateString(hinterface, disp_flag); // update string // update displaying items srtuct if(disp_flag) hinterface->Displaying.Regs.all ¦= (1<<(i+NOTHING_SELECTED)); else hinterface->Displaying.Regs.all &= ~(1<<(i+NOTHING_SELECTED)); } return hinterface->disp_cnt; } /** * @brief Print string with menu item. * @param hinterface - handle for interface. * @param SELECTED_DEFINE - define for choosing print string as currently selected, or as regular. * @param Print_Function - function to print string with item to char array @ref Print_PWM_Sine_Hz. * @return disp_flag - flag is string displaying or not. * @note This call @ref Print_PWM_Sine_Hz or smth function like that. */ uint8_t LCD_PrintString(Interface_HandleTypeDef *hinterface, unsigned SELECTED_DEFINE, void (*Print_Function)(Interface_HandleTypeDef *, uint8_t)) { uint8_t disp_flag = 0; hinterface->disp_cnt += 1; if(hinterface->disp_cnt > 0) { disp_flag = 1; if (hinterface->CurrentSelection == SELECTED_DEFINE) Print_Function(hinterface, 1); else Print_Function(hinterface, 0); } else { disp_flag = 0; } return disp_flag; } /** * @brief Update string. * @param hinterface - handle for interface. * @param disp_flag - flag to display string or not. * @return disp_flag - flag is string displaying or not. * @note For I2C display: Display string. * @note For SPI display: print in frame buffer. Display update happens in Update_LCDDisplay. */ uint8_t LCD_UpdateString(Interface_HandleTypeDef *hinterface, uint8_t disp_flag) { // if there something to display if(hinterface->disp_cnt <= hinterface->hlcd.LCD_Rows) { if(disp_flag) { #ifdef LCD_I2C LCD_SetString(&hinterface->hlcd, hinterface->disp_cnt-1); LCD_Send_STRING(&hinterface->hlcd, hinterface->LCD_String); #endif #ifdef LCD_SPI static uint8_t line = 0; // reset line to zero if its first item displaying if (hinterface->disp_cnt == 1) line = 0; SPIDisp_Output_String(0, line, hinterface->LCD_String, 0, 0); line += 10; #endif } } else { disp_flag = 0; } return disp_flag; } //------------------PRINT FUNCTIONS-----------------// void Print_PWM_Sine_Hz(Interface_HandleTypeDef *hinterface, uint8_t selected_flag) { if(selected_flag) snprintf(hinterface->LCD_String, hinterface->hlcd.LCD_Width+1, ">Sine: %d.%02d Hz", pwm_ctrl[R_PWM_CTRL_PWM_SINE_HZ]/100, pwm_ctrl[R_PWM_CTRL_PWM_SINE_HZ]%100); else snprintf(hinterface->LCD_String, hinterface->hlcd.LCD_Width+1, "Sine: %d.%02d Hz", pwm_ctrl[R_PWM_CTRL_PWM_SINE_HZ]/100, pwm_ctrl[R_PWM_CTRL_PWM_SINE_HZ]%100); } void Print_PWM_Duty(Interface_HandleTypeDef *hinterface, uint8_t selected_flag) { if(selected_flag) snprintf(hinterface->LCD_String, hinterface->hlcd.LCD_Width+1, ">Duty: %d.%02d%%", pwm_ctrl[R_PWM_CTRL_DUTY_BRIDGE]/100, pwm_ctrl[R_PWM_CTRL_DUTY_BRIDGE]%100); else snprintf(hinterface->LCD_String, hinterface->hlcd.LCD_Width+1, "Duty: %d.%02d%%", pwm_ctrl[R_PWM_CTRL_DUTY_BRIDGE]/100, pwm_ctrl[R_PWM_CTRL_DUTY_BRIDGE]%100); } void Print_PWM_Hz(Interface_HandleTypeDef *hinterface, uint8_t selected_flag) { if(selected_flag) snprintf(hinterface->LCD_String, hinterface->hlcd.LCD_Width+1, ">PWM: %d Hz", pwm_ctrl[R_PWM_CTRL_PWM_HZ]); else snprintf(hinterface->LCD_String, hinterface->hlcd.LCD_Width+1, "PWM: %d Hz", pwm_ctrl[R_PWM_CTRL_PWM_HZ]); } void Print_PWM_MaxPulseDur(Interface_HandleTypeDef *hinterface, uint8_t selected_flag) { #ifdef LCD_I2C if(hpwm1.stim.sTickBasePrescaler > 1) { if(selected_flag) snprintf(hinterface->LCD_String, hinterface->hlcd.LCD_Width+1, ">Max: %d us (%d)", hpwm1.PWM_MaxPulseDur/hpwm1.stim.sTickBasePrescaler, hpwm1.PWM_MaxPulseDur); else snprintf(hinterface->LCD_String, hinterface->hlcd.LCD_Width+1, "Max: %d us (%d)", hpwm1.PWM_MaxPulseDur/hpwm1.stim.sTickBasePrescaler, hpwm1.PWM_MaxPulseDur); } else { if(selected_flag) snprintf(hinterface->LCD_String, hinterface->hlcd.LCD_Width+1, ">Max: %d us", hpwm1.PWM_MaxPulseDur); else snprintf(hinterface->LCD_String, hinterface->hlcd.LCD_Width+1, "Max: %d us", hpwm1.PWM_MaxPulseDur); } #endif #ifdef LCD_SPI if(hpwm1.stim.sTickBasePrescaler > 1) { if(selected_flag) snprintf(hinterface->LCD_String, hinterface->hlcd.LCD_Width+1, ">Max dur: %d us (%d)", (uint32_t)hpwm1.PWM_MaxPulseDur/hpwm1.stim.sTickBasePrescaler, hpwm1.PWM_MaxPulseDur); else snprintf(hinterface->LCD_String, hinterface->hlcd.LCD_Width+1, "Max dur: %d us (%d)", (uint32_t)hpwm1.PWM_MaxPulseDur/hpwm1.stim.sTickBasePrescaler, hpwm1.PWM_MaxPulseDur); } // else if(hpwm1.stim.sTickBasePrescaler > 10) // { // if(selected_flag) // snprintf(hinterface->LCD_String, hinterface->hlcd.LCD_Width+1, ">Max dur: %d 10ns (%d)", (uint32_t)hpwm1.PWM_MaxPulseDur*100/hpwm1.stim.sTickBasePrescaler, hpwm1.PWM_MaxPulseDur); // else // snprintf(hinterface->LCD_String, hinterface->hlcd.LCD_Width+1, "Max dur: %d 10ns (%d)", (uint32_t)hpwm1.PWM_MaxPulseDur*100/hpwm1.stim.sTickBasePrescaler, hpwm1.PWM_MaxPulseDur); // } // else if(hpwm1.stim.sTickBasePrescaler > 1) // { // if(selected_flag) // snprintf(hinterface->LCD_String, hinterface->hlcd.LCD_Width+1, ">Max dur: %d 100ns (%d)", (uint32_t)hpwm1.PWM_MaxPulseDur*10/hpwm1.stim.sTickBasePrescaler, hpwm1.PWM_MaxPulseDur); // else // snprintf(hinterface->LCD_String, hinterface->hlcd.LCD_Width+1, "Max dur: %d 100ns (%d)", (uint32_t)hpwm1.PWM_MaxPulseDur*10/hpwm1.stim.sTickBasePrescaler, hpwm1.PWM_MaxPulseDur); // } else { if(selected_flag) snprintf(hinterface->LCD_String, hinterface->hlcd.LCD_Width+1, ">Max dur: %d us", hpwm1.PWM_MaxPulseDur); else snprintf(hinterface->LCD_String, hinterface->hlcd.LCD_Width+1, "Max dur: %d us", hpwm1.PWM_MaxPulseDur); } #endif } void Print_PWM_MinPulseDur(Interface_HandleTypeDef *hinterface, uint8_t selected_flag) { #ifdef LCD_I2C if(hpwm1.stim.sTickBasePrescaler > 1) { if(selected_flag) snprintf(hinterface->LCD_String, hinterface->hlcd.LCD_Width+1, ">Min: %d ns", (uint32_t)hpwm1.PWM_MinPulseDur*1000/hpwm1.stim.sTickBasePrescaler); else snprintf(hinterface->LCD_String, hinterface->hlcd.LCD_Width+1, "Min: %d ns", (uint32_t)hpwm1.PWM_MinPulseDur*1000/hpwm1.stim.sTickBasePrescaler); } else { if(selected_flag) snprintf(hinterface->LCD_String, hinterface->hlcd.LCD_Width+1, ">Min: %d us", hpwm1.PWM_MinPulseDur); else snprintf(hinterface->LCD_String, hinterface->hlcd.LCD_Width+1, "Min: %d us", hpwm1.PWM_MinPulseDur); } #endif #ifdef LCD_SPI if(hpwm1.stim.sTickBasePrescaler > 1) { if(selected_flag) snprintf(hinterface->LCD_String, hinterface->hlcd.LCD_Width+1, ">Min dur: %d us (%d)", (uint32_t)hpwm1.PWM_MinPulseDur/hpwm1.stim.sTickBasePrescaler, hpwm1.PWM_MinPulseDur); else snprintf(hinterface->LCD_String, hinterface->hlcd.LCD_Width+1, "Min dur: %d us (%d)", (uint32_t)hpwm1.PWM_MinPulseDur/hpwm1.stim.sTickBasePrescaler, hpwm1.PWM_MinPulseDur); } // else if(hpwm1.stim.sTickBasePrescaler > 10) // { // if(selected_flag) // snprintf(hinterface->LCD_String, hinterface->hlcd.LCD_Width+1, ">Min dur: %d 10ns (%d)", (uint32_t)hpwm1.PWM_MinPulseDur*100/hpwm1.stim.sTickBasePrescaler, hpwm1.PWM_MinPulseDur); // else // snprintf(hinterface->LCD_String, hinterface->hlcd.LCD_Width+1, "Min dur: %d 10ns (%d)", (uint32_t)hpwm1.PWM_MinPulseDur*100/hpwm1.stim.sTickBasePrescaler, hpwm1.PWM_MinPulseDur); // } // else if(hpwm1.stim.sTickBasePrescaler > 1) // { // if(selected_flag) // snprintf(hinterface->LCD_String, hinterface->hlcd.LCD_Width+1, ">Min dur: %d 100ns (%d)", (uint32_t)hpwm1.PWM_MinPulseDur*10/hpwm1.stim.sTickBasePrescaler, hpwm1.PWM_MinPulseDur); // else // snprintf(hinterface->LCD_String, hinterface->hlcd.LCD_Width+1, "Min dur: %d 100ns (%d)", (uint32_t)hpwm1.PWM_MinPulseDur*10/hpwm1.stim.sTickBasePrescaler, hpwm1.PWM_MinPulseDur); // } else { if(selected_flag) snprintf(hinterface->LCD_String, hinterface->hlcd.LCD_Width+1, ">Min dur: %d us", hpwm1.PWM_MinPulseDur); else snprintf(hinterface->LCD_String, hinterface->hlcd.LCD_Width+1, "Min dur: %d us", hpwm1.PWM_MinPulseDur); } #endif } void Print_PWM_DeadTime(Interface_HandleTypeDef *hinterface, uint8_t selected_flag) { #ifdef LCD_I2C if(selected_flag) snprintf(hinterface->LCD_String, hinterface->hlcd.LCD_Width+1, ">DeadT: %d us", pwm_ctrl[R_PWM_CTRL_DEAD_TIME]); else snprintf(hinterface->LCD_String, hinterface->hlcd.LCD_Width+1, "DeadT: %d us", pwm_ctrl[R_PWM_CTRL_DEAD_TIME]); #endif #ifdef LCD_SPI if(selected_flag) snprintf(hinterface->LCD_String, hinterface->hlcd.LCD_Width+1, ">Dead Time: %d us", pwm_ctrl[R_PWM_CTRL_DEAD_TIME]); else snprintf(hinterface->LCD_String, hinterface->hlcd.LCD_Width+1, "Dead Time: %d us", pwm_ctrl[R_PWM_CTRL_DEAD_TIME]); #endif } void Print_PWM_TickBasePresc(Interface_HandleTypeDef *hinterface, uint8_t selected_flag) { #ifdef LCD_I2C if(selected_flag) snprintf(hinterface->LCD_String, hinterface->hlcd.LCD_Width+1, ">Tick Presc: %d", pwm_ctrl[R_PWM_CTRL_TICKBASE_PRESCALER]); else snprintf(hinterface->LCD_String, hinterface->hlcd.LCD_Width+1, "Tick Presc: %d", pwm_ctrl[R_PWM_CTRL_TICKBASE_PRESCALER]); #endif #ifdef LCD_SPI if(selected_flag) snprintf(hinterface->LCD_String, hinterface->hlcd.LCD_Width+1, ">TickBase Prescaler: %d", pwm_ctrl[R_PWM_CTRL_TICKBASE_PRESCALER]); else snprintf(hinterface->LCD_String, hinterface->hlcd.LCD_Width+1, "TickBase Prescaler: %d", pwm_ctrl[R_PWM_CTRL_TICKBASE_PRESCALER]); #endif } void Print_PWM_DC(Interface_HandleTypeDef *hinterface, uint8_t selected_flag) { if(selected_flag) { if(hpwm1.sPWM_Config.PWM_Mode->DC) snprintf(hinterface->LCD_String, hinterface->hlcd.LCD_Width+1, ">Mode: DC"); else snprintf(hinterface->LCD_String, hinterface->hlcd.LCD_Width+1, ">Mode: Sine"); } else { if(hpwm1.sPWM_Config.PWM_Mode->DC) snprintf(hinterface->LCD_String, hinterface->hlcd.LCD_Width+1, "Mode: DC"); else snprintf(hinterface->LCD_String, hinterface->hlcd.LCD_Width+1, "Mode: Sine"); } } void Print_PWM_BRIDGE(Interface_HandleTypeDef *hinterface, uint8_t selected_flag) { if(selected_flag) { if(hpwm1.sPWM_Config.PWM_Mode->BRIDGE) snprintf(hinterface->LCD_String, hinterface->hlcd.LCD_Width+1, ">Bridge: Bridge"); else snprintf(hinterface->LCD_String, hinterface->hlcd.LCD_Width+1, ">Bridge: Single"); } else { if(hpwm1.sPWM_Config.PWM_Mode->BRIDGE) snprintf(hinterface->LCD_String, hinterface->hlcd.LCD_Width+1, "Bridge: Bridge"); else snprintf(hinterface->LCD_String, hinterface->hlcd.LCD_Width+1, "Bridge: Single"); } } void Print_PWM_PHASE(Interface_HandleTypeDef *hinterface, uint8_t selected_flag) { if(selected_flag) { if(hpwm1.sPWM_Config.PWM_Mode->PHASE) snprintf(hinterface->LCD_String, hinterface->hlcd.LCD_Width+1, ">Phase: 3-Phase"); else snprintf(hinterface->LCD_String, hinterface->hlcd.LCD_Width+1, ">Phase: One Phase"); } else { if(hpwm1.sPWM_Config.PWM_Mode->PHASE) snprintf(hinterface->LCD_String, hinterface->hlcd.LCD_Width+1, "Phase: 3-Phase"); else snprintf(hinterface->LCD_String, hinterface->hlcd.LCD_Width+1, "Phase: One Phase"); } } void Print_PWM_POLARITY(Interface_HandleTypeDef *hinterface, uint8_t selected_flag) { if(selected_flag) { if(hpwm1.sPWM_Config.PWM_Mode->POLARITY) snprintf(hinterface->LCD_String, hinterface->hlcd.LCD_Width+1, ">Polarity: Neg"); else snprintf(hinterface->LCD_String, hinterface->hlcd.LCD_Width+1, ">Polarity: Pos"); } else { if(hpwm1.sPWM_Config.PWM_Mode->POLARITY) snprintf(hinterface->LCD_String, hinterface->hlcd.LCD_Width+1, "Polarity: Neg"); else snprintf(hinterface->LCD_String, hinterface->hlcd.LCD_Width+1, "Polarity: Pos"); } } void Print_PWM_ACTIVECHANNEL(Interface_HandleTypeDef *hinterface, uint8_t selected_flag) { if(selected_flag) snprintf(hinterface->LCD_String, hinterface->hlcd.LCD_Width+1, ">PWM channel: %d", hpwm1.sPWM_Config.PWM_Mode->ACTIVECHANNEL); else snprintf(hinterface->LCD_String, hinterface->hlcd.LCD_Width+1, "PWM channel: %d", hpwm1.sPWM_Config.PWM_Mode->ACTIVECHANNEL); } /** * @brief First initialization of interface struct and necessary peripheral. * @note This called from StartMainTask */ void EncoderFirstInit(void) { hinterface.DoubleClick_Timeout = 200; // tim settings TIM_ENCODER.htim.Instance = TIMER_ENCODER_INSTANCE; TIM_ENCODER.htim.Init.Period = 0xFFFF; TIM_ENCODER.htim.Init.Prescaler = 0; TIM_ENCODER.sTimMode = TIM_IT_MODE; // TIM_ENCODER.sTickBaseUS = TIMER_ENCODER_TICKBASE; // TIM_ENCODER.sTimAHBFreqMHz = TIMER_ENCODER_AHB_FREQ; // TIM_ENCODER.sTimFreqHz = 0; TIM_Base_Init(&TIM_ENCODER); hinterface.henc.sConfig.EncoderMode = TIM_ENCODERMODE_TI2; hinterface.henc.sConfig.IC1Polarity = TIM_INPUTCHANNELPOLARITY_FALLING; hinterface.henc.sConfig.IC1Selection = TIM_ICSELECTION_DIRECTTI; hinterface.henc.sConfig.IC1Prescaler = TIM_ICPSC_DIV1; hinterface.henc.sConfig.IC1Filter = 5; hinterface.henc.sConfig.IC2Polarity = TIM_INPUTCHANNELPOLARITY_FALLING; hinterface.henc.sConfig.IC2Selection = TIM_ICSELECTION_DIRECTTI; hinterface.henc.sConfig.IC2Prescaler = TIM_ICPSC_DIV1; hinterface.henc.sConfig.IC2Filter = 5; hinterface.henc.GPIOx = TIMER_ENCODER_PORT; hinterface.henc.GPIO_PIN_TI1 = TIMER_ENCODER_PIN1; hinterface.henc.GPIO_PIN_TI2 = TIMER_ENCODER_PIN2; hinterface.henc.GPIO_PIN_SW = TIMER_ENCODER_PIN_SW; TIM_Encoder_Init(&hinterface.henc, &TIM_ENCODER.htim); HAL_TIM_Base_Start(&TIM_ENCODER.htim); } /** * @brief First initialization of SPI LCD. * @note This called from Interface_FirstInit */ void SPILCD_FirstInit(void) { sspi_disp.hspi.Instance = SPI1; sspi_disp.hspi.Init.Mode = SPI_MODE_MASTER; sspi_disp.hspi.Init.Direction = SPI_DIRECTION_2LINES; sspi_disp.hspi.Init.DataSize = SPI_DATASIZE_8BIT; sspi_disp.hspi.Init.CLKPolarity = SPI_POLARITY_LOW; sspi_disp.hspi.Init.CLKPhase = SPI_PHASE_1EDGE; sspi_disp.hspi.Init.NSS = SPI_NSS_SOFT; sspi_disp.hspi.Init.BaudRatePrescaler = SPI_BAUDRATEPRESCALER_4; sspi_disp.hspi.Init.FirstBit = SPI_FIRSTBIT_MSB; sspi_disp.hspi.Init.TIMode = SPI_TIMODE_DISABLE; sspi_disp.hspi.Init.CRCCalculation = SPI_CRCCALCULATION_DISABLE; sspi_disp.CLK_GPIOx = GPIOA; sspi_disp.CLK_PIN = GPIO_PIN_5; sspi_disp.MISO_GPIOx = GPIOA; sspi_disp.MISO_PIN = GPIO_PIN_6; sspi_disp.MOSI_GPIOx = GPIOA; sspi_disp.MOSI_PIN = GPIO_PIN_7; SPI_Base_Init(&sspi_disp); SPIDisp_Init(); } /** * @brief First initialization of LCD (SPI/I2C corresponding to define). * @note This called from Interface_FirstInit */ void LCD_FirstInit(void) { hinterface.LCD_prevUpdateTick = xTaskGetTickCount(); #ifdef LCD_I2C hinterface.hlcd.hi2c = &hi2c1; hinterface.LCD_ClearDelay = 10; hinterface.LCD_UpdateDelay = 250; LCD_Init(&hinterface.hlcd, NumbOfLines, NumbOfSymb); #endif #ifdef LCD_SPI hinterface.hlcd.hspi = &sspi_disp.hspi; hinterface.hlcd.LCD_Width = NumbOfSymb; hinterface.hlcd.LCD_Rows = NumbOfLines; hinterface.LCD_UpdateDelay = 100; SPILCD_FirstInit(); #endif } /** * @brief First initialization of Encoder Timer. * @note This called from Interface_FirstInit */ void Interface_FirstInit(void) { LCD_FirstInit(); EncoderFirstInit(); }

# ПРИЛОЖЕНИЕ Г

Листинг interface.h

/** ************************************************************************** * @file interface.h * @brief Заголовочный файл модуля реализации интерфейса (енкодер и LCD дисплей). * * @defgroup INTERFACE * @brief Interface stuff * *************************************************************************/ #ifndef __INTERFACE_H_ #define __INTERFACE_H_ #include "periph_general.h" #include "i2c_lcd.h" #include "gmg12864_r9ofg.h" //#define LCD_I2C #define LCD_SPI #ifdef LCD_I2C #define NumbOfSymb 16 #define NumbOfLines 2 #define LCD_HandleTypeDef LCDI2C_HandleTypeDef #endif #ifdef LCD_SPI #define NumbOfSymb 30 #define NumbOfLines 6 #define LCD_HandleTypeDef LCDSPI_HandleTypeDef #endif /** * @addtogroup INTERFACE_MENU_DEFINES * @ingroup INTERFACE * @brief Defines for menu * @{ */ /** * @brief Struct for "selected" values for each coils for PWM. */ typedef enum { NOTHING_SELECTED, /*!< nothing selected to change */ PWM_SINE_HZ_SELECTED, /*!< SINE HZ selected to change */ PWM_DUTY_SELECTED, /*!< DUTY selected to change */ PWM_HZ_SELECTED, /*!< PWM HZ selected to change */ PWM_MAXPULSE_SELECTED, /*!< MAX PULSE selected to change */ PWM_MINPULSE_SELECTED, /*!< MIN PULSE DC selected to change */ PWM_DEADTIME_SELECTED, /*!< DEADTIME selected to change */ PWM_TICKBASEPRESC_SELECTED, /*!< TICKBASEPRESCALER selected to change */ }CurrentSelectionRegs_TypeDef; /** * @brief Struct for "selected" values for each coils for PWM. */ typedef enum { NOTHING_COIL_SELECTED, /*!< nothing selected to change */ PWM_COIL_DC_SELECTED, /*!< COIL DC selected to change */ PWM_COIL_BRIDGE_SELECTED, /*!< COIL BRIDGE selected to change */ PWM_COIL_PHASE_SELECTED, /*!< COIL PHASE selected to change */ PWM_COIL_POLARITY_SELECTED, /*!< COIL POLARITY selected to change */ PWM_COIL_ACTIVECHANNEL_SELECTED, /*!< COIL ACTIVECHANNEL selected to change */ }CurrentSelectionCoil_TypeDef; /** * @brief Struct with indexes for each menu. */ typedef enum { REGISTERS_MENU = 0, /*!< Registers menu is choosed */ COILS_MENU, /*!< Coils menu is choosed */ }MenuIndex_TypeDef; /** * @brief Struct for registers menu of settings for PWM. * @details If item is showed - bit is setted. If item isnt showed - bit is reseted */ typedef union { uint32_t all; /*!< Value to access all bits at once */ /** * @brief Struct of PWM params with separated bits */ struct { unsigned PWM_Sine_Hz:1; unsigned PWM_Duty:1; unsigned PWM_Hz:1; unsigned PWM_MaxPulseDur:1; unsigned PWM_MinPulseDur:1; unsigned PWM_DeadTime:1; unsigned PWM_TickBasePresc:1; }Items; }RegsMenu_HandleTypeDef; /** * @brief Struct for coils menu of settings for PWM. * @details If item is showed - bit is setted. If item isnt showed - bit is reseted */ typedef union { uint32_t all; /*!< Value to access all bits at once */ /** * @brief Struct of PWM params with separated bits */ struct { unsigned PWM_DC:1; unsigned PWM_BRIDGE:1; unsigned PWM_PHASE:1; unsigned PWM_POLARITY:1; unsigned PWM_ACTIVECHANNEL:1; }Items; }CoilsMenu_HandleTypeDef; /** * @brief Struct with all settings for PWM: coils and registers. */ typedef struct { RegsMenu_HandleTypeDef Regs; /*!< PWM parametrs in modbus registers */ CoilsMenu_HandleTypeDef Coils; /*!< PWM parametrs in modbus coils */ }Menu_TypeDef; #define Menu_Regs_Size 7 ///< size of @ref RegsMenu_HandleTypeDef #define Menu_Coil_Size 5 ///< size of @ref RegsMenu_HandleTypeDef #define Menu_PWM_Sine_Hz (1<<0) ///< position of PWM_Sine_Hz in @ref RegsMenu_HandleTypeDef #define Menu_PWM_Duty (1<<1) ///< position of PWM_Duty in @ref RegsMenu_HandleTypeDef #define Menu_PWM_Hz (1<<2) ///< position of PWM_Hz in @ref RegsMenu_HandleTypeDef #define Menu_PWM_MaxPulseDur (1<<3) ///< position of PWM_MaxPulseDur in @ref RegsMenu_HandleTypeDef #define Menu_PWM_MinPulseDur (1<<4) ///< position of PWM_MinPulseDur in @ref RegsMenu_HandleTypeDef #define Menu_PWM_DeadTime (1<<5) ///< position of PWM_DeadTime in @ref RegsMenu_HandleTypeDef #define Menu_PWM_TickBasePresc (1<<6) ///< position of PWM_TickBasePresc in @ref RegsMenu_HandleTypeDef #define Menu_PWM_DC (1<<0) ///< position of PWM_DC in @ref CoilsMenu_HandleTypeDef #define Menu_PWM_BRIDGE (1<<1) ///< position of PWM_BRIDGE in @ref CoilsMenu_HandleTypeDef #define Menu_PWM_PHASE (1<<2) ///< position of PWM_PHASE in @ref CoilsMenu_HandleTypeDef #define Menu_PWM_POLARITY (1<<3) ///< position of PWM_POLARITY in @ref CoilsMenu_HandleTypeDef #define Menu_PWM_ACTIVECHANNEL (1<<4) ///< position of PWM_ACTIVECHANNEL in @ref CoilsMenu_HandleTypeDef /** INTERFACE_MENU_DEFINES * @} */ /** * @addtogroup INTERFACE_HANDLE_DEFINES * @ingroup INTERFACE * @brief Defines for menu * @{ */ typedef struct { LCD_HandleTypeDef hlcd; TIM_EncoderTypeDef henc; // menu vars Menu_TypeDef Menu; Menu_TypeDef Displaying; uint32_t CurrentSelection; MenuIndex_TypeDef MenuNumber; unsigned FineChanging:1; int8_t StartDispItem; int8_t disp_cnt; // lcd vars char LCD_String[NumbOfSymb+1]; portTickType LCD_prevUpdateTick; uint16_t LCD_UpdateDelay; uint8_t LCD_ClearDelay; // encoder vars uint32_t Encoder_Shdw; uint32_t Switch_Shdw; uint32_t Switch_prevTick; uint32_t DoubleClick_Timeout; }Interface_HandleTypeDef; extern Interface_HandleTypeDef hinterface; /** INTERFACE_HANDLE_DEFINES * @} */ //--------------------------------FUNCTIONS---------------------------------- /** * @addtogroup INTERFACE_FUNCTIONS * @ingroup INTERFACE * @brief Function for controling interface and menu */ //-----------------ENCODER FUNCTIONS----------------// /** * @addtogroup INTERFACE_ENCODER_FUNCTIONS * @ingroup INTERFACE_FUNCTIONS * @brief Function for controling encoder * @{ */ /* Update Encoder */ void Update_Encoder(Interface_HandleTypeDef *hinterface); /* Start Single Click Actions */ void Encoder_SingleClick(Interface_HandleTypeDef *hinterface); /* Start Double Click Actions */ void Encoder_DoubleClick(Interface_HandleTypeDef *hinterface); /* Holding Switch action */ uint8_t Encoder_HoldingSwitch(Interface_HandleTypeDef *hinterface); /* Holding Switch and Rotate encoder action */ uint8_t Encoder_HoldingAndRotateEncoder(Interface_HandleTypeDef *hinterface); /* Release Switch action */ void Encoder_SwitchRelease(Interface_HandleTypeDef *hinterface); /* Update params variables from encoder rotate */ void Encoder_UpdateVarsFromEncoder(Interface_HandleTypeDef *hinterface); /* Set uint16_t value from from encoder rotate */ void Encoder_SetUint16Value(TIM_EncoderTypeDef *henc, uint16_t *TargetValue, uint16_t coef, int32_t min_value, int32_t max_value); /* Set uint8_t value from from encoder rotate */ void Encoder_SetUint8Value(TIM_EncoderTypeDef *henc, uint8_t *TargetValue, uint16_t coef, int32_t min_value, int32_t max_value); /** INTERFACE_ENCODER_FUNCTIONS * @} */ //---------------LCD DISPLAY FUNCTIONS--------------// /** * @addtogroup INTERFACE_LCD_FUNCTIONS * @ingroup INTERFACE_FUNCTIONS * @brief Function for controling lcd display * @{ */ /* Update Active Items in Menu Structure for displaying */ void Update_ActiveMenuItems(Interface_HandleTypeDef *hinterface); /* Update LCD display */ void Update_LCDDisplay(Interface_HandleTypeDef *hinterface); /* Display all coils menu at LCD Display */ uint8_t LCD_DisplayMenuCoils(Interface_HandleTypeDef *hinterface); /* Display all registers menu at LCD Display */ uint8_t LCD_DisplayMenuRegisters(Interface_HandleTypeDef *hinterface); /* Print string with menu item */ uint8_t LCD_PrintString(Interface_HandleTypeDef *hinterface, unsigned SELECTED_DEFINE, void (*Print_Function)(Interface_HandleTypeDef *, uint8_t)); /* Update string */ uint8_t LCD_UpdateString(Interface_HandleTypeDef *hinterface, uint8_t disp_flag); /** INTERFACE_LCD_FUNCTIONS * @} */ //------------------PRINT FUNCTIONS-----------------// /** * @addtogroup INTERFACE_PRINT_FUNCTIONS * @ingroup INTERFACE_FUNCTIONS * @brief Function for print string in char array. * @param hinterface - handle for interface * @param selected_flag - flag to print string as currently selected, or as regular * @note Char array: hinterface->LCD_String * @{ */ void Print_PWM_Sine_Hz(Interface_HandleTypeDef *hinterface, uint8_t selected_flag); void Print_PWM_Hz(Interface_HandleTypeDef *hinterface, uint8_t selected_flag); void Print_PWM_Duty(Interface_HandleTypeDef *hinterface, uint8_t selected_flag); void Print_PWM_MaxPulseDur(Interface_HandleTypeDef *hinterface, uint8_t selected_flag); void Print_PWM_MinPulseDur(Interface_HandleTypeDef *hinterface, uint8_t selected_flag); void Print_PWM_DeadTime(Interface_HandleTypeDef *hinterface, uint8_t selected_flag); void Print_PWM_TickBasePresc(Interface_HandleTypeDef *hinterface, uint8_t selected_flag); void Print_PWM_DC(Interface_HandleTypeDef *hinterface, uint8_t selected_flag); void Print_PWM_BRIDGE(Interface_HandleTypeDef *hinterface, uint8_t selected_flag); void Print_PWM_PHASE(Interface_HandleTypeDef *hinterface, uint8_t selected_flag); void Print_PWM_POLARITY(Interface_HandleTypeDef *hinterface, uint8_t selected_flag); void Print_PWM_ACTIVECHANNEL(Interface_HandleTypeDef *hinterface, uint8_t selected_flag); /** INTERFACE_PRINT_FUNCTIONS * @} */ //--------------INITIALIZATION FUNCTIONS------------// /** * @addtogroup INTERFACE_INIT_FUNCTIONS * @ingroup INTERFACE_FUNCTIONS * @brief Function for initialize interface and menu * @{ */ /* First initialization of interface struct and necessary peripheral */ void Interface_FirstInit(void); /* First initialization of SPI LCD */ void SPILCD_FirstInit(void); /* First initialization of LCD (SPI/I2C corresponding to define) */ void LCD_FirstInit(void); /* First initialization of Encoder Timer */ void EncoderFirstInit(void); /** INTERFACE_INIT_FUNCTIONS * @} */ #endif // __INTERFACE_H_

# ПРИЛОЖЕНИЕ Д

Листинг control.c

/** ************************************************************************** * @file control.c * @brief Модуль для управления ШИМ и записи логов. *************************************************************************/ #include "pwm.h" TIM_SettingsTypeDef TIM_CTRL = {0}; // variables for filling arrays int Numb_Of_Peroids = 2; // number of periods int Samples_Per_Peroid = 0; // how many samples in one period int Size_Of_Log = 0; // size of written data to log int log_ind = 0; // index of log arrays int cnt_to_cnt_log = 0; // counter for log_cnt int sine_ind_prev = 0; /** * @brief Filling logs. * @note Заполнение логов: синус, шим, пила. * @note This called from TIM_CTRL_Handler */ void Fill_Logs_with_Data(void) { // calc pwm duty from timer float PWM_Duty; if(PWM_Get_Mode(&hpwm1, PWM_DC_MODE) == 0) // if sinus need to be written { if(PWM_Get_Mode(&hpwm1, PWM_BRIDGE_MODE)) // if its signed sine mode (two channels) { if(hpwm1.Duty_Table_Ind < hpwm1.Duty_Table_Size/2) // first half get from channel 1 PWM_Duty = (((float)PWM_Get_Compare1(&hpwm1))/(PWM_Get_Autoreload(&hpwm1)))+1; else // second half get from channel 2 PWM_Duty = 1-(((float)PWM_Get_Compare2(&hpwm1))/(PWM_Get_Autoreload(&hpwm1))); } else // if its unsigned sine mode (single channel) { // just get current pwm duty PWM_Duty = ((float)PWM_Get_Compare1(&hpwm1)/PWM_Get_Autoreload(&hpwm1)); } } else // if its dc pwm mode { // just get current pwm duty if(PWM_Get_Mode(&hpwm1, PWM_BRIDGE_MODE)) // if its second channels mode PWM_Duty = ((float)PWM_Get_Compare2(&hpwm1)/PWM_Get_Autoreload(&hpwm1)); else // if its first channel mode PWM_Duty = ((float)PWM_Get_Compare1(&hpwm1)/PWM_Get_Autoreload(&hpwm1)); } // WRITE SINUS TO WHOLE ARRAY // sine_log[log_ind] = sin_val; if(PWM_Get_Mode(&hpwm1,PWM_DC_MODE) == 0) // in table mode write PWM Duty (write sine) with scale 1/2 from sin table max value (0xFFFF/2) sine_log[log_ind] = PWM_Duty*(0x8000-1); else // in dc mode write PWM Duty (write sine) sine_log[log_ind] = 0; // WRITE PWM if(PWM_Get_Mode(&hpwm1,PWM_DC_MODE)) // in DC mode { // write 1 - if log_ind < Size_Of_Period*PWM_Dury // write 0 - otherwise pwm_log[log_ind] = (log_ind%(Size_Of_Log/Numb_Of_Peroids) < (Size_Of_Log/Numb_Of_Peroids+1)*hpwm1.PWM_Sine_Hz/100)? 1: 0; } else // in table mode { // write fill whole pwm array at one interrupt int PWM_Period_End_Ind = (Size_Of_Log/Numb_Of_Peroids); int PWM_Step_End_Ind; if(PWM_Get_Mode(&hpwm1,PWM_BRIDGE_MODE)) PWM_Step_End_Ind = PWM_Period_End_Ind*fabs(PWM_Duty-1); else PWM_Step_End_Ind = PWM_Period_End_Ind*PWM_Duty; for(int i = 0; i <= PWM_Step_End_Ind; i++) { for (int j = 0; j < Numb_Of_Peroids; j++) pwm_log[i+j*PWM_Period_End_Ind] = 1; } for(int i = PWM_Step_End_Ind+1; i < PWM_Period_End_Ind; i++) for (int j = 0; j < Numb_Of_Peroids; j++) pwm_log[i+j*PWM_Period_End_Ind] = 0; } // WRITE COUNTER cnt_log[log_ind] = (uint16_t)(hpwm1.PWM_Sine_Hz*100); cnt_to_cnt_log++; if(cnt_to_cnt_log>=Size_Of_Log/2) cnt_to_cnt_log = 0; // INCREMENT AND RESET COUNTER log_ind++; if(PWM_Get_Mode(&hpwm1,PWM_DC_MODE) == 0) // if its PWM table mode { // SYNCHRONIZE PERIOD OF SIN IN LOG // (это надо, чтобы данные не съезжали из-за несинхронизированного периода) // wait until period ended if(log_ind>Size_Of_Log-1) // if logs are filled { if((unsigned)hpwm1.Duty_Table_Ind < sine_ind_prev) // and if new period started { log_ind = 0; // reset counter sine_ind_prev = (unsigned)hpwm1.Duty_Table_Ind; } } // update prev variable only if log currently writing else sine_ind_prev = (unsigned)hpwm1.Duty_Table_Ind; } else // if its PWM DC mode { // if logs are filled if(log_ind>Size_Of_Log-1) log_ind = 0; } // if its overflow log array size - reset log_ind if(log_ind>LOG_SIZE-1) { log_ind = 0; sine_ind_prev = (unsigned)hpwm1.Duty_Table_Ind; } } /** * @brief Update log parameters. * @note Проверка надо ли обновлять параметры логов, и если надо - обновляет их. * @note This called from TIM_CTRL_Handler */ void Update_Params_For_Log(void) { unsigned UpdateLog = 0; // READ NUMB OF PERIOD IN LOGS if(Numb_Of_Peroids != log_ctrl[R_LOG_CTRL_LOG_PWM_NUMB]) { Numb_Of_Peroids = log_ctrl[R_LOG_CTRL_LOG_PWM_NUMB]; // update logs params UpdateLog = 1; } // READ SIZE OF LOGS if(Size_Of_Log != log_ctrl[R_LOG_CTRL_LOG_SIZE]) { Size_Of_Log = log_ctrl[R_LOG_CTRL_LOG_SIZE]; // update logs params UpdateLog = 1; } // UPDATE LOG PARAMS if(UpdateLog) { // set logs params Set_Log_Params(); } } /** * @brief Set up log parameters. * @note Устанавливает настройки логов и проверяет их на корректность. */ void Set_Log_Params(void) { // SET LOG PARAMS log_ind = 0; Samples_Per_Peroid = TIM_CTRL.sTimFreqHz/hpwm1.PWM_Sine_Hz; if(Size_Of_Log > LOG_SIZE) // if its too much data in log { Numb_Of_Peroids = (LOG_SIZE/Samples_Per_Peroid); log_ctrl[R_LOG_CTRL_LOG_SIZE] = Numb_Of_Peroids; Size_Of_Log = Numb_Of_Peroids*Samples_Per_Peroid; } // clear logs arrays for(int i = Size_Of_Log; i < LOG_SIZE; i++) { sine_log[i] = 0; pwm_log[i] = 0; cnt_log[i] = 0; } } /** * @brief reInitialization of control timer. * @note Перенастраивает таймер согласно принятным настройкам в log_ctrl. * @note This called from main while */ void Control_Timer_ReInit(TIM_SettingsTypeDef *stim) { TIM_Base_MspDeInit(&stim->htim); hpwm1.stim.sTickBaseUS = PROJSET_MEM->TIM_CTRL_TICKBASE; TIM_Base_Init(stim); HAL_TIM_Base_Start_IT(&stim->htim); // timer for sinus HAL_NVIC_SetPriority(TIM8_BRK_TIM12_IRQn, 1, 1); } /** * @brief First initialization of Control Timer. * @note Первый управляющего таймера. Таймер записывает логи и обновляет параметры ШИМ. * @note This called from main */ void Control_Timer_FirstInit(void) { //-------CONTROL TIMER INIT---------- // tim settings TIM_CTRL.htim.Instance = TIMER_CTRL_INSTANCE; TIM_CTRL.sTimMode = TIM_IT_MODE; TIM_CTRL.sTickBaseUS = PROJSET.TIM_CTRL_TICKBASE; TIM_CTRL.sTimAHBFreqMHz = PROJSET.TIM_CTRL_AHB_FREQ; TIM_CTRL.sTimFreqHz = HZ_TIMER_CTRL; TIM_Base_Init(&TIM_CTRL); HAL_NVIC_SetPriority(TIM8_TRG_COM_TIM14_IRQn, 1, 1); HAL_TIM_Base_Start_IT(&TIM_CTRL.htim); // timer for sinus // FILL TIME ARRAY WITH TIME for(int i = 0; i < R_TIME_LOG_QNT; i++) time_log[i] = i; } //------------------------------------------------------------------- //------------------------HANDLERS FUNCTIONS------------------------- //-------------CONTROL TIMER--------------- void TIM8_UP_TIM13_IRQHandler(void) { /* TIM_CTRL_Handler */ Trace_CTRL_TIM_Enter(); HAL_TIM_IRQHandler(&TIM_CTRL.htim); Fill_Logs_with_Data(); Update_Params_For_Log(); PWM_Update_Params(&hpwm1); WriteSettingsToMem(); Trace_CTRL_TIM_Exit(); }

# ПРИЛОЖЕНИЕ Е

Листинг control.h

/** ************************************************************************** * @file control.h * @brief Заголовочный файл для модуля управления ШИМ и записи логов. *************************************************************************/ #ifndef __CONTROL_H_ #define __CONTROL_H_ #include "periph_general.h" #include "modbus.h" #include "math.h" //#include "settings.h" #define M_PI 3.14159265358979323846 /* pi */ extern TIM_SettingsTypeDef TIM_CTRL; //---------------------this called from TIM_CTRL_Handler()----------------------- /** * @brief Update log parameters. * @note Проверка надо ли обновлять параметры логов, и если надо - обновляет их. * @note This called from TIM_CTRL_Handler */ void Update_Params_For_Log(void); /** * @brief Filling logs. * @note заполнение логов: синус, шим, пила. * @note this called from TIM_CTRL_Handler */ void Fill_Logs_with_Data(void); /** * @brief Set up log parameters. * @note Устанавливает настройки логов и проверяет их на корректность. */ void Set_Log_Params(void); // this called from main while(1) /** * @brief reInitialization of control timer. * @param stim - указатель на настройки таймера. * @note Перенастраивает таймер согласно принятным настройкам в log_ctrl. * @note This called from main while */ void Control_Timer_ReInit(TIM_SettingsTypeDef *stim); /** * @brief First initialization of Control Timer. * @note Первый управляющего таймера. Таймер записывает логи и обновляет параметры ШИМ. * @note This called from main */ void Control_Timer_FirstInit(void); /** * @brief First initialization of Encoder Timer. * @note This called from main */ void EncoderFirstInit(void); #endif // __CONTROL_H_